## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Control Engineering(4219) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List 3 differences between open loop and closed loop control systems. |  |
| b) | Explain the effect of feedback on overall gain of a system |  |
| c) | Explain any one application of synchros in brief. |  |
| d) | Draw the block diagram representation of a DC Tachometer and state its Transfer function. |  |
| e) | List any two standard test signals and explain the same. |  |
| f) | Write short note on system stability. |  |
| g) | Define system Poles and Zeroes. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram, explain a closed loop system. |  |
| b) | Explain any one practical example of a Feedback Control System. |  |
| c) | By means of Routh Criterion, determine the stability of the system represented by the following characteristic equation. 3s4 + 10s3 + 5s2 + 5s + 2 = 0 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain the working of a separately excited DC generator. State the relevant expressions. |  |
| b) | Explain the working and characteristics of an AC Servomotor. |  |
| c) | What is a shaft encoder? List the different types. Explain any one in detail. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For the circuit shown in fig.1, find the transfer function. |  |
| b) | Define the following terms and give expressions for the same: i) Direct Transmission Gain ii) Open Loop Transfer Function iii) Closed Loop Transfer Function. |  |
| c) | Draw block diagram representation for a potentiometer. Determine its Transfer Function. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A step input position Servomechanism system has the following constants: R=1, K=6, J=1, h=1, F=5. Find m(t). |  |
| b) | For a system with loop gain HG(s)=50s(s+10) and R(s)=5s. Find the steady state error. |  |
| c) | Explain Type 0, Type 1 and Type 2 systems. Compare the steady state errors for the same. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch Bode plot for the magnitude of the function G(s) = 10s |  |
|  |  | …..2/- |
|  | -2- |  |
| b) | A fourth order system is represented by the following characteristic equation s4 + 8s3 + 18s2 + 16s + 5 = 0. Using Routh Criterion, determine the stability of the system. |  |
| c) | Given the transfer function G(s)=100ss+10, determine the gain from Bode Plot. |  |