## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-II(4062) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the terms ‘Pitching’, ‘Swaying’ and ‘Bouncing’. |  |
| b) | What do you mean by ‘Transmission Efficiency’? What should be its range? |  |
| c) | What is banking of Roads and why it is done? |  |
| d) | What do you mean by monocoque construction? |  |
| e) | State the condition for True Rolling of wheels during steering. |  |
| f) | What is ‘Independent Suspension’? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a road performance curve and explain the term economical speed. |  |
| b) | Explain ‘Rolling Resistance’. How can it be minimised? |  |
| c) | A 4-wheeler has a weight of 10,000 N. Its engine develops a power of 40.2 KW at 6000 rpm. Its gear ratio is 15:1 in the first gear and axle ratio is 1.5:1, transmission efficiency in the first gear is 90%, wheel diameter is 80 cm. Calculate the vehicle speed and tractive force at the wheels. Neglect the resistances. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a recirculating ball steering gear box. |  |
| b) | Explain with neat sketches what are ‘Camber’, ‘Caster’ and ‘King pin inclination’ and their effect on handling of steering. |  |
| c) | i) List the functions of the steering system. ii) Explain the Ackerman steering mechanism. | (3) (3) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a sketch of a Variator and explain its working. |  |
| b) | With sketches explain the working of a Fluid Flywheel. |  |
| c) | Explain one type of Epicyclic gear train used in car transmission. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the MacPherson strut suspension and enumerate its main advantages. |  |
| b) | With a neat sketch explain the construction and working of a trailing link suspension. |  |
| c) | What is Air Suspension? Distinguish between conventional and Air Suspensions. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How is an automobile painting done in an Industry? Explain with a sketch. |  |
| b) | With the help of a neat, proportionate sketch explain the working of a seat reclining mechanism of a car. |  |
| c) | Explain ‘Front visibility’, its importance and factors involved. |  |

***********