## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg./A

Subject: Thermal Engineering(ME303) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam Tables and Mollier’s charts is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by the term property of a system? Briefly classify the properties of a system. |  |
| b) | Define internal energy and state its units. |  |
| c) | State and explain Kelvin-Planck statement of second law of Thermodynamics. |  |
| d) | State and explain Avogadro’s law. |  |
| e) | Explain what is meant by critical pressure ratio of a nozzle. |  |
| f) | State and explain Dalton’s law of partial pressure. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define and represent the following processes on P-V and T-S diagram: i) Polytropic process ii) Isobaric process. |  |
| b) | Determine the mass of oxygen in a cylinder of 0.2m3 capacity at a pressure of 100 bar and temperature of 17C. Take universal gas constant as 8.314KJ/Kg-mol-K. |  |
| c) | 0.2 Kg of air at a pressure of 105 KPa and temperature 15ºC is compressed isentropically to a pressure of 2.835 KPa. Calculate the final temperature of the gas, change in internal energy and work done on the gas during the compression process. =1.41 Cv = 0.703 KJ/KgK |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find out the internal energy of 1Kg of steam at a pressure of 8 bar when the system is superheated, the degree of superheat being 100ºC. Take Cp = 2.093 KJ/KgK |  |
| b) | In a Rankine cycle, the maximum pressure of steam supplied is 6 bar. The dryness fraction is 0.9. The exhaust pressure is 0.7 bar. Using Mollier’s chart, find Rankine efficiency. |  |
| c) | With help of a neat sketch, explain the construction and working of a steam calorimeter. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of Cornish boiler, with help of a neat sketch. |  |
| b) | With a neat sketch, explain the construction and working of a Benson boiler. |  |
| c) | What is the purpose of a fusible plug? Explain its construction and working with help of a neat sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of a reaction type of a steam turbine. |  |
| b) | Describe with a neat sketch induced draught cooling tower. |  |
| c) | A heat engine working at the rate of 105 KW has an efficiency of 22%. Determine the quantity of heat received and transferred from working fluid. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Indian Boiler Regulations |  |
| b) | Compounding of Turbines |  |
| c) | Surface condenser |  |
| d) | Irreversibility |  |

***********