## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./EC/MED/EI/COMP/E/E&EE

### Subject: Basic Electronics(4132) ][Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the energy band diagram for a semiconductor and explain. |  |
| b) | Compare the CE, CB and CC configurations of a transistor. |  |
| c) | List the important JFET parameters. |  |
| d) | Explain ‘Secondary Electron Emission’. |  |
| e) | Explain the concept ‘Thermal Runway’ with regards to Transistor. |  |
| f) | Define dc and βdc of a transistor. Also give its equations. Draw necessary diagram. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram, explain the working of n-type semiconductor. |  |
| b) | Draw and explain the V-I characteristics of a p-n junction diode. |  |
| c) | With neat diagram and waveforms, explain the working of a half wave rectifier. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For a full wave rectifier, show that the ripple factor is 0.482 and its rectification efficient is 81%. |  |
| b) | With a neat diagram, explain the working of zener diode. Also draw its V-I characteristics. |  |
| c) | A 10V zener diode is used to regulate the voltage across a variable load resistance. The i/p voltage varies between 13V and 16V. The load current varies between 10mA and 85mA. Minimum zener current is 15mA. Calculate the value of series resistance R. Refer figure (a). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram, explain the construction and working of P-N-P transistor. |  |
| b) | Draw and explain the input-output characteristics for a transistor in CE configuration. |  |
| c) | Draw and explain the Collector to Base bias circuit. Derive necessary equations for the same. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by transistor biasing? Explain the need for transistor biasing. What are the requirements of a biasing circuit? |  |
| b) | Calculate the value of the three currents. Refer figure (b). |  |
| c) | For the circuit in figure (c), calculate the dc bias voltage and current. Assume VBE=0.3V and β = 60. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short notes on: i) Cathode materials ii) Thermionic emission. |  |
| b) | Explain the construction and working of DE-MOSFET. | ……2/- |
|  | -2- |  |
| c) | Explain the operation and output characteristics of p-channel JFET. |  |