## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Elementary Ship Design(5331) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Design features of passenger ship |  |
| b) | Owner’s requirements |  |
| c) | Preliminary design |  |
| d) | Displacement equation |  |
| e) | Sectional area curve |  |
| f) | GRT and NRT |  |
| g) | Machinery spaces |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the various constraints on dimensions in ship design. |  |
| b) | How and when is detail design carried out in ship design? Explain in context with design spiral. |  |
| c) | Explain how the machinery weight of vessel can be estimated in the preliminary design. |  |
| Q.No.3. | Estimate the main dimensions, dead weight, displacement at a 30m long fishing trawler with endurance of 14 days and for a catch of 20 tonnes. The total power of the vessel is 490 HP. The fuel consumption is at the rate of 155 gms/BHP/hour. The oil consumption is 3 gms/BHP/hour. For the stipulated endurance, provision required is 0.15 tonne/person, fresh water required is 1tonne/person, crew and effects is 0.15 tonne/person. Total number of crew is 20 given that: dwt/ = 0.22; CB = 0.5 - 0.54; L/B = 4.2 to 4.5; B/T = 2.5 to 2.68; T/D = 0.7 | (12) |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Describe in brief ‘Lines plan’ for a ship. |  |
| b) | How sectional area curve can be obtained? |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A ship in loaded condition has a displacement of 16420 tonnes, draft 7.4m, CB = 0.66, CW = 0.767. If the length of the ship is L = 120m, estimate the TPC and MCT at this loaded condition. Also estimate the draft for a displacements of 8800 tonnes. |  |
| b) | Draw a typical GA plan of a cargo vessel. Name and explain the various important spaces in the plan. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | How is the minimum freeboard requirement of a vessel determined? Explain the effects of freeboard on the design of a vessel. |  |
| b) | Explain the specifications and contract between the ship owner and shipbuilder. |  |

***********