## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engg.

Subject: Hydraulic Machinery(4053) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the terms (i) Compressibility (ii) Surface Tension. |  |
| b) | Calculate mass density, specify weight and specific gravity of a liquid which weighs 12N in a volume of 1 litre. |  |
| c) | Express intensity of pressure of 1 Kgf/cm2 in terms of ‘metres of water’. |  |
| d) | State ‘Pascal’s law’ and its two practical applications. |  |
| e) | State ‘Bernoullis Theorem’. State any two assumptions made in the derivation of Bernoullis equation. |  |
| f) | State the laws of fluid friction. |  |
| g) | Define pressure Head and state its unit. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A differential manometer connected to a pipe at two points A & B which are at the same level containing oil of specific gravity of 0.8 shows difference in mercury level of 9cm. Draw neat sketch and determine difference in pressure at the two points. |  |
| b) | As isosceles triangular plate of base 3m and altitude 3.5m is immersed vertically in a oil of specific gravity 0.9 such that the base of the plate coincides with the free surface of the oil. Determine the total pressure and centre of pressure on the plate face. |  |
| c) | With the help of neat sketch explain the construction and working of ‘Bourdan’s Tube pressure gauge’. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 200mm x 100mm Venturimeter is provided in a vertical pipe carrying water flowing in a upward direction. A differential mercury manometer is connected to the inlet and the throat gives reading of 210mm. Find the rate of flow. Assume coefficient of discharge is equals to 0.98. |  |
| b) | A pipe 300m long has a slope of 1 in 100 and tapers from 1m diameter at high end to 0.50m at low end. The quantity of water flowing is 5400 m3/min. If the pressure at the high end is 49 KN/m2, find the pressure at the lower end. |  |
| c) | Explain the types of flow in the pipes. Draw neat sketch of pitot tube to measure velocity of a liquid and explain. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular notch 1.6m wide carries water with a head of 0.9m. Find the discharge. If the same discharge passes over a 60 ‘V’ notch, find the head over this notch. Assume Cd = 0.62 for both the notches. |  |
| b) | A vertical sharp-edged orifice 120mm diameter is discharging water at the rate of 0.0982 m3/s under a constant head of 10m. A point on the jet measured from the venacontracta of the jet has co-ordinates 4.5m horizontal and 0.54m vertical. Find co-efficient of velocity and coefficient of discharge. | …….2/- |
|  | -2- |  |
| c) | Write a short note on the following: - Air vessels in reciprocating pump and its functions - Priming in centrifugal pumps |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch, explain the construction and working of hydraulic jack. |  |
| b) | With the help of neat labelled sketch, explain the construction and working of centrifugal pump. |  |
| c) | A single acting reciprocating pump running at 50 rpm delivers 0.007 m3/sec of water. The diameter of the piston is 200mm and stroke length is 300mm. The suction and the delivery heads are 3.5m and 12m respectively. Determine (i) Theoretical discharge (ii) Slip of the pump (iii) Power required to run the pump. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a neat labelled sketch of external gear pump and explain its working. |  |
| b) | Draw a neat labelled sketch of vane pump and state its two applications. |  |
| c) | List any three operational difficulties commonly experienced in centrifugal pump and their remedies to overcome the same. |  |
| d) | State four differences between centrifugal pump and reciprocating pump. |  |

***********