## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(CE301) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain scope of surveying. |  |
| b) | Draw the conventional sign for the following: i) Embankment ii) Road iii) Stream. |  |
| c) | Explain various instruments used to set perpendiculars. |  |
| d) | Define the following terms: i) Level line ii) Bench mark. |  |
| e) | What precautions should be taken to eliminate the errors in leveling? |  |
| f) | Explain principle of plane table survey. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain indirect ranging with neat sketch. |  |
| b) | What are the errors encounted in chaining? Also explain the precautions and obstacles in chaining. |  |
| c) | Explain local attraction. State the reasons and errors caused due to local attraction. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain chain triangulation, stating selection of survey stations and base line. |  |
| b) | Following are the bearing of the closed traverse. Calculate interior angle of the traverse. Line; FB; BB. AB; 120º30'; 300º30'. BC; 192º45'; 12º45'. CD; 280º00'; 100º00'. DA; 45º30'; 225º30'. |  |
| c) | The following bearings were observed in a compass traverse. Line; FB; BB. AB; 140º45'; 318º15'. BC; 216º30'; 38º00'. CD; 209º15'; 30º15'. DE; 319º45'; 139º45'. EA; 60º15'; 240º15'. |  |
|  | At which of these stations would local attraction be suspected? Find the corrected bearings of the lines. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short note on classification of survey. |  |
| b) | The following consecutive readings were taken with a levelling instrument at an interval of 15m: 2.130, 1.750, 0.610, 3.255, 2.825, 2.040, 1.870, 0.995, 0.345, 1.610, 2.355 and 3.730. The instrument was shifted after fourth and eight reading. The first reading was taken on BM of RL 110.00m. Calculate RL’s of all the points by collimation system. |  |
|  |  | …….2/- |
|  | -2- |  |
| c) | The following successive reading were taken with a dumpy level along a chain line at common interval of 20m. The first reading was taken on a chainage of 110m. The RL of second change point was 102.125m. Instrument was shifted after third and seventh reading. Calculate RLs of all the points by rise and fall method. 3.150, 2.285, 1.105, 3.870, 2.220, 0.760, 2.320, 0.470, 1.935, 3.115 and 3.870m. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the merits and demerits of plane table survey? |  |
| b) | Describe briefly permanent adjustment of dumpy level. |  |
| c) | Explain intersection method of plane table surveying. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Differential levelling |  |
| b) | Classification of survey |  |
| c) | Chaining on sloping ground |  |
| d) | Quadrantal bearing system |  |

***********