## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT

Subject: Soil Mechanics(4012) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following: i) Moisture content ii) Density index (LD). |  |
| b) | Explain how the knowledge of soil mechanics helps in foundation design and selection. |  |
| c) | Define flow lines and equipotential lines. |  |
| d) | State the difference between compaction and consolidation. |  |
| e) | Define the term ‘unconformity’ & enlist the types. |  |
| f) | Explain ‘bore hole logs’. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the term void ratio and porosity and derive the relationship between them. |  |
| b) | One cubic metre of wet soil weighs 22KN and has the specific gravity 2.7 and moisture content 14%. Find void ratio and dry unit weight. |  |
| c) | Explain in detail the sedimentation analysis. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail IS classification of soil. |  |
| b) | Enlist and explain any three factors affecting permeability of soil. |  |
| c) | A falling head permeameter contains a soil sample of 8 cm high and 60cm2 is the cross sectional area. The permeability of the sample is expected to be 1x10-4 cm/s. If it is desired that the head in the stand pipe to fall from 30cm to 10cm in 40 minutes, find the size of the stand pipe, which should be used. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with aid of neat sketch the tri-axial test. |  |
| b) | Explain the plate load test and its limitations. |  |
| c) | Enlist the objects of soil exploration. Also explain electric resistivity method. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch, explain the Folds: i) Anticlines – Folds ii) Synclines – Folds iii) Fan Folds. |  |
| b) | Explain sand replacement method to find field density of soil. |  |
| c) | Explain any three methods of field compactions. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Cement stabilization |  |
| b) | Spring analogy. |  |
| c) | Coulomb’s theory of earth pressure |  |
| d) | Drained and undrained test |  |
| e) | Applications of geology in civil Engineering |  |

***********