## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-I(4054) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between SI and CI engine. |  |
| b) | Define i) Piston Displacement ii) Clearance Volume. |  |
| c) | During idling of an SI engine a rich air-fuel mixture is required. Why? |  |
| d) | State the functions of a carburetor. |  |
| e) | List the additives added to petrol and briefly explain their uses. |  |
| f) | Explain at least 3 types of fans for IC engine water cooling. |  |
| g) | List the factors on which heat dissipated in any engine cooling depends. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following data refers to a test on a four stroke cycle oil engine: Number of Cylinders=4, Cylinder Bore=9cm, Piston stroke=13cm, Speed=2100rpm, Brake torque=167N-m, Fuel oil consumption=13.5 liters/hr, Colorific value of oil=44000 KJ/Kg, Specific Gravity of oil = 0.82 Calculate i) Brake mean effective pressure (b.m.e.p) ii) The Brake thermal efficiency. |  |
| b) | Compare 2 stroke and 4 stroke engines. |  |
| c) | What is brake power? Describe the calculation of Brake power using a neatly drawn Rope Brake Dynamometer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch the refining of petroleum by Fractional distillation. |  |
| b) | List the advantages of a MPFI engine over a carburetion engine. |  |
| c) | Explain with a sketch the construction and working of a mechanical fuel pump, highlighting its drawbacks. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with sketches i) Radiator ii) Thermostat. |  |
| b) | With the help of a neat sketch explain the construction and working of Bosch type fuel injector. |  |
| c) | Explain with sketches the Diesel Injection Nozzles and their specific uses. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch showing an oil breather on an IC engine. State its functions. Also explain in brief, what is meant by i) Dilution of lubrication oil ii) Crankcase ventilation. |  |
| b) | What is meant by pre-ignition and how is it distinguished from detonation? |  |
| c) | Describe the different types of SI engine combustion chambers with the help of sketches. | …..2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the objectives of IC engine lubrication? List the requirements of a good engine lubricant and describe them. |  |
| b) | Draw the valve timing diagram for 4-stroke SI engine and describe the same. Why does suction process start before TDC? |  |
| c) | Draw a neat labelled sketch to show the constructional features of a typical piston. State the functions of different parts. |  |

***********