## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Welding in Ship Construction-II(4337) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between TIG and MIG welding. |  |
| b) | What is the use of jigs and fixtures in welding? |  |
| c) | Write short note on ‘leak test’. |  |
| d) | Briefly summarize the important welding symbols. |  |
| e) | What is the purpose of estimating the cost of weld? |  |
| f) | List the protective devices used during welding. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch the submerged arc welding process. |  |
| b) | Discuss the importance of Pre-heat treatment and Post-heat treatment in welding. |  |
| c) | What are the problems faced while welding aluminium? How they can be overcome? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different methods adopted for welding dissimilar metals? Briefly explain any one method. |  |
| b) | Describe the step-by-step procedure to carry out Dye-penetrate test to locate welding defects. |  |
| c) | Explain the x-ray test as carried out in shipyards. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | As per the Indian Register of shipping explain the welding procedure qualification test. |  |
| b) | With a simple sketch explain slot welding. |  |
| c) | Calculate the total cost of manual flux shielded metal arc welding using following data: Welding length : 12m Welding speed : 12m/hr Operating factor : 30% Labour & overhead charges : Rs.25/hr Electrode consumption : 3.5 Kg/m Arc Voltage : 22 Volts Arc Current : 200 Amps E = 0.6 Electricity charges : 1.20 per KW-hr. Electrode price : Rs.200/Kg |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the method of metal spraying in maintenance welding. |  |
| b) | Distinguish between cladding and hard facing. |  |
| c) | Electrode on safe welding practices. |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Welding stainless steel |  |
| b) | Bend test in welding |  |
| c) | Welding positions |  |
| d) | Weld tacking |  |

***********