## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg./FT/A/SB/MNE

Subject: Elements of Electrical & Electronics Engg.(CS302) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a single line diagram, showing the voltage levels in the various stages in the flow of electrical power from 110KV substation to 11KV/400V distribution transformer. |  |
| b) | Define the following i) Average value ii) R.M.S. value of an ac wave. |  |
| c) | Write a short note on ‘colour codes of single phase and three phase PVC cables’. |  |
| d) | List out any three applications of DC series and DC shunt motor. |  |
| e) | What is the necessity of starter in a three phase induction motor? |  |
| f) | What is P-N junction diode? How potential barrier is formed in P-N junction diode? |  |
| g) | With the help of neat circuit diagram, explain the working of full wave centre-tap rectifier. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain ‘Plate Earthing’ with neat diagram. |  |
| b) | i) Define Earthing and explain its necessity. ii) What is double Earthing? What are its advantages? | (4) (2) |
| c) | i) Explain concealed conduit wiring system. State its 2 advantages and 2 disadvantages. ii) Give two applications of concealed conduit wiring system. | (5) (1) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) List out specifications of PVC cables. (six points) ii)Explain the importance of providing fuse in electrical circuit. | (3) (3) |
| b) | i) With the help of neat diagram explain the construction of HRC fuse. ii)State any two advantages of HRC fuses over rewirable fuses. | (4) (2) |
| c) | Draw the symbol and state the function of i) Fuse switch unit ii) MCB iii) MCCB. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain different types of losses occurring in core and windings of transformer. Also explain how these losses can be minimized. |  |
| b) | i) Explain the significance of KVA rating of transformer. ii) Define Transformer. Explain what is all day efficiency of a transformer. | (3) (3) |
| c) | With the help of a neat diagram, explain the construction of DC motor. |  |
|  |  | ……..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Explain the working principle of Dc motor. ii) Explain the function of ‘Commutator’ in a DC motor. | (4) (2) |
| b) | With the help of neat diagram, explain the construction of stator and rotor of three phase squirrel cage induction motor. |  |
| c) | i) State the various types of starters used for 3 phase induction motors, indicating clearly the motor for which it is used. ii) How do you reverse the direction of rotation of three phase induction motor? | (4) (2) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) With the help of neat circuit diagram and waveforms explain the working of full wave bridge rectifier. ii) State any two merits of full wave bridge rectifier over centre-tap rectifier. | (4) (2) |
| b) | Draw the symbols and truth tables of following gates: i) EX-NOR ii) OR iii) NOT. |  |
| c) | With the help of neat diagram & graph, explain the V-I characteristics of a P-N junction diode. |  |

***********