## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg./E&EE

### Subject: A.C. Machines(4113)/Electrical Machines-II(EE401) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain why induction motor cannot run at synchronous speed. |  |
| b) | Define i) Starting torque ii) Pull out torque. |  |
| c) | Draw the power flow diagram of induction motor. |  |
| d) | What is the need of a starter while starting a 3 induction motor? |  |
| e) | Explain ‘hunting’ in alternator. |  |
| f) | State need for parallel operation of alternators. What are the conditions that must be satisfied before connecting two alternators in parallel? |  |
| g) | Explain working principle of Reluctance motor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the production of rotating magnetic field in 3 phase induction motor. |  |
| b) | Draw and explain phasor diagram of induction motor operating at full load condition. |  |
| c) | Explain torque slip characteristics of 3 phase induction motor on load. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 50HZ, 8 pole Induction motor has full load slip of 4%. The rotor resistance and standstill reactance are 0.01 and 0.1 per phase respectively. Find i) speed at which torque is maximum ii) ratio of full load torque to maximum torque. |  |
| b) | With a neat power and control circuit diagram, explain working of DOL starter used for starting 3 induction motor. |  |
| c) | Explain the construction and working of alternator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of shaded pole motor. |  |
| b) | Write a note on capacitor start and run 1 induction motor. |  |
| c) | Discuss synchronizing of 3 alternator by i) Three dark lamp method ii) Synchroscope method. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain armature reaction at unity and lagging p.f. loads in an alternator. |  |
| b) | Draw and explain phasor diagram of alternator at no load and on load condition. |  |
| c) | Write a note on calculation of synchronous impedance by open circuit and short circuit test. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the special features of synchronous motors? Mention its applications. |  |
| b) | Discuss the effect of varying excitation on armature current and power factor in synchronous motor. |  |
| c) | Write a note on hysteresis motor. |  |

***********