## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Computer Numerical Control Machines(ME628) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of CNC code table permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define automation in manufacturing industry. |  |
| b) | List the three basic components of NC system. |  |
| c) | What is meant by target point? |  |
| d) | Which is the most commonly used program format? Give the general form. |  |
| e) | Name the common techniques used for repetitive programming. |  |
| f) | What is meant by DOF in Robot? |  |
| g) | What is meant by fixed zero? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify NC machines in various groups with respect to spindle and work piece. |  |
| b) | Which are the modules used to assemble the NC machine tool? |  |
| c) | How to set zero point at any position on the machine table. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain open-loop-control system in NC/CNC machines. |  |
| b) | Explain why word format system is used in part programming. |  |
| c) | Explain with example preparatory function GOO. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain point-to-point NC/CNC control system. |  |
| b) | Machining is to be done at AB, BC, CD and DA. Write the CNC part program for the job given in Fig.1. Thickness of the plate is 15mm. |  |
| c) | Explain tool length compensation. Give example. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a part program for the turning operation to be performed on the job shown in Fig.2, where the diameter is to be reduced to 30mm from the available bar of 40mm diameter. Maximum depth of cut being limited to 3mm. |  |
| b) | It is required to mill a square pocket 30 x 30mm at various positions on a flat plat. The following information is available. Depth of profile = 3mm Z = 0 is at the surface of the flat plate. The tool should retract back to a position 5mm above plate surface while moving position. The co-ordinates of starting point of position are A(50,50), B(120,120), C(200,200). Write the part program using subroutine. |  |
| c) | Write note on APT language structure. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a part program to drill holes using fixed cycle. The size of drill is 10. The reference plane is 2mm above the surface of plat. The plate thickness is 15mm. The size of the plate is 150 x 150mm. The through holes are to be drilled at A and B position, whose coordinates are A(25,35) and B(75,70). Assume over travel of 3mm for drilling operation. |  |
| b) | Write the APT geometry and motion statements necessary to perform the drilling portion of the job in Fig.3. Set the plane 5mm above the part surface. The job is 15mm thick |  |
| c) | Write note on Qualified tools. |  |