## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg./E/FT

Subject: Refrigeration & Airconditioning(5034)/(ME502) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Coefficient Of Performance (COP). State its importance. |  |
| b) | State the reason why a refrigerant needs to possess, - Low specific volume of vapour - Low specific heat of liquid. |  |
| c) | Compare the vapour compression system with the vapour absorption system. |  |
| d) | State the advantages and disadvantages of the hermetically sealed compressor. |  |
| e) | Define: i) Psychrometry ii) Specific humidity. |  |
| f) | What is Room sensible heat factor? State its significance. |  |
| g) | Explain the following Psychrometric processes with the help of Psychrometric chart: i) sensible cooling ii) sensible heating iii) cooling & humidification. |  |
| h) | List down the commonly used refrigerants in the cascade refrigeration system. State the applications of the cascade refrigeration system. (Only for students of revised scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Represent the vapour compression cycle on the T-S and P-H diagrams stating clearly the processes involved. |  |
| b) | Giving neat sketch explain the construction and working of a refrigeration system based on the vapour compression cycle. |  |
| c) | Write a note on R-22. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Giving neat sketch explain the construction and working of a domestic Electrolux refrigerator. |  |
| b) | Giving neat sketch explain the construction and working of a thermostatic expansion valve. |  |
| c) | Differentiate between: i) Air cooled and water cooled condensers ii) Natural and forced draft cooling towers |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the construction and working of a window air conditioner. |  |
| b) | What is humidity control? Giving neat sketch explain the working of any one humidistat. |  |
| c) | Write a note on ‘Air handling unit’. |  |
| d) | List down the various Air conditioning components & controls & explain any two of them. (Only for students of old scheme) |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List down the applications of a sling psychrometer. Further, giving neat sketch explain its construction and working. |  |
| b) | Describe and represent the following processes on the psychrometric chart: - Mixing of air streams - Cooling and dehumidification |  |
| c) | Write a note on ‘Types of supply air outlets’. |  |
| Q.No.6. | Write notes on any four: | 4 x 3 = 12 |
| a) | Standard rating of a refrigerating machine |  |
| b) | Applications of vapour absorption system |  |
| c) | Psychrometric chart |  |
| d) | GSHF |  |
| e) | Classification of refrigeration systems |  |
| f) | Central Air conditioning plant (Only for students of old scheme) |  |

***********