## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./COMP/EC/EI

Subject: Basic Electrical Engineering(CS304) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the waveform of sinusoidal AC voltage and indicate therein the following terms with formula: - Peak Value (ii) RMS Value (iii) Average Value. |  |
| b) | Briefly explain the concept & significance of ‘Phase sequence’ in 3 AC systems. |  |
| c) | List any 6 methods of reducing earth resistance. |  |
| d) | Write the emf equation of a transformer. State and explain the factors on which the induced emf depends. |  |
| e) | State the working principle of a DC motor. Also briefly explain the process of energy conversion therein. |  |
| f) | State three points of distinction between squirrel cage and slip ring induction motor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain the star and delta connection in 3 AC system. Also state the relation between the line and phase quantities of each type. |  |
| b) | With a neat labelled diagram explain plate earthing. |  |
| c) | List 6 factors influencing earthing resistance. Also state any 3 IE rules relevant to earthing. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define a transformer. With neat diagram explain its working principle. State any 2 of its applications. |  |
| b) | State the two types of transformers based upon the construction of the core. Compare the two types. |  |
| c) | Write short note on following: i) Losses in transformer ii) Voltage regulation in transformer. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the various parts of a DC motor. State the functions of its 4 main parts. |  |
| b) | Explain the classification of DC motors based upon the method of connection of field and armature winding. Draw neat diagrams of each type. |  |
| c) | Briefly explain the term starter and its necessity for starting a DC motor. Explain the method for reversal of direction of rotation of a DC shunt motor. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working principle of a three phase induction motor. With the help of neat diagrams explain the production of revolving flux on application of 3 supply. |  |
| b) | List the different types of starters that can be used for starting a 3 squirrel cage induction motor. With neat diagram explain the construction and working of any one of these types of starters. | ……2/- |
|  | -2- |  |
| c) | With the help of neat diagram explain the reason why 1 induction motor is not starting. Also explain the principle behind split phase starting methods. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define a fuse. List the types of fuse. Briefly explain the terms: i) breaking capacity ii) rupturing capacity of a fuse. |  |
| b) | With neat diagram explain the construction, working and functions of a MCB. |  |
| c) | Write a short note on current operated ELCB. |  |

***********