## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Computer Engg./E&EE

Subject: Introduction to Microcontrollers(CO512) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe three differences between microprocessors and microcontrollers. |  |
| b) | Describe the math flags of µc 8051. |  |
| c) | Describe any two arithmetic instructions of µc 8051. |  |
| d) | Given A = 73 H and RO = F5H, what are the contents of A and RO after the execution of each of these instructions: (i) XRL A, RO (ii) ANL A, RO |  |
| e) | Draw a schematic showing µc 8051 connected to a single common–anode seven segment LED display. |  |
| f) | List any six features of an ATMEGA series (AVR) microcontroller. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a diagram, describe the circuit of port 1 of µc 8051. Also explain how data is read from the port pin. |  |
| b) | Draw a schematic diagram showing connections of external memory (ROM and RAM) to µc 8051. |  |
| c) | Draw and describe the format of TCON register of µc 8051. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is addressing mode? Explain three addressing modes of µc 8051 with an example of each. |  |
| b) | Describe the following instructions of µc 8051 with examples: (i) MUL (ii) DIV |  |
| c) | Write a program to transfer a block of ten data bytes, which are stored in RAM locations from 40H onward, into RAM locations starting at 50H. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of schematic diagram, describe the connection of a 2 x 2 matrix keyboard to the µc 8051. |  |
| b) | Draw and describe the connections of an 8–bit ADC to the µc 8051. |  |
| c) | Describe six differences between the µc 8051 and PIC µc. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe any three bit-manipulation instructions with an example of each. |  |
| b) | Given SP=30H and contents of RAM locations 30H=11H, 2FH = 22H, 50H=33H and 31H=45H; what is the outcome when µc 8051 executes (i) PUSH 50H (ii) POP 50H Write values of contents of all the above RAM locations and SP. |  |
| c) | Explain the factors on which the selection of a microcontroller of an embedded system depends. |  |
| Q.No.6. | Answer the following Questions: | 3 x 4 = 12 |
| a) | Write a note on interfacing an LCD to µc 8051. |  |
| b) | Explain the increment and decrement instructions of µc 8051 that use direct addressing mode. | ……2/- |
|  | -2- |  |
| c) | Calculate the time delay obtained when timer/counter 0 is used as a timer in mode 2. THO=86 H and the XTAL frequency is 12 MHz. |  |
|  | OR |  |
| c) | Draw a block diagram and describe Harvard architecture. |  |

***********