## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Fluid Power Control(ME627) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any three applications of fluid power. |  |
| b) | What is meant by FRL unit? |  |
| c) | What is meant by shuttle valve? |  |
| d) | What is a Telescopic cylinder? When is it normally used? |  |
| e) | What is the difference between Flarrred and Flareless pipe joints? |  |
| f) | Give three probable causes for excessive noise in a pneumatic system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State any six desirable properties of a hydraulic fluid. |  |
| b) | Compare hydraulic system with pneumatic system. (any four points) |  |
| c) | Explain the basic construction of a flexible hose. Give any three factors to be considered while installing a flexible hose. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by Quick exhaust valve? Illustrate its use giving a circuit. |  |
| b) | Describe the construction and working of spool type 4/2 way Direction Control Valve and give its symbol. |  |
| c) | What is the function of a sequence valve? Describe its construction & working giving a sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is cylinder cushioning? State its purpose. Describe the cushioning operation giving a sketch. |  |
| b) | An hydraulic cylinder is required to apply a force of 40000N during extension. If the pump generates a pressure of 100 bar, find - Required piston Diameter - Piston rod diameter if the force during return stroke is half the force during extension stroke. |  |
| c) | Draw regenerative hydraulic circuit and explain its working. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Show the use of accumulator in a hydraulic circuit and explain the circuit. |  |
| b) | Draw a pneumatic circuit to illustrate ‘AND’ gate. Give any one application of ‘AND’ gate. |  |
| c) | Draw a pneumatic circuit to show the use of time delay valve and explain it. |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Compressed air piping layout |  |
| b) | Hydraulic intensifier |  |
| c) | Speed control of cylinder in pneumatics |  |
| d) | Troubleshooting of hydraulic system |  |

***********