## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg.

### Subject: Testing & Maintenance Of Electrical Equipments(5104)/

Testing & Maintenance Of Electrical Machines(EE613) [old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is meant by type test, routine test & acceptance test of electrical equipment? |  |
| b) | Justify the statement: “Generally large capacity machines are tested by indirect method of testing”. |  |
| c) | Define i) Mechanical efficiency ii) Electrical efficiency iii) Overall efficiency for a d.c. machine. |  |
| d) | Distinguish between Brake test & Swinburne’s test performed on dc machines. (3points). |  |
| e) | State the various methods of revarnishing of winding of electrical equipments. |  |
| f) | What do you mean by regenerative testing? State its advantages & disadvantages. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enlist various losses occurring in a dc machine as per IS classification. Also state the parameters on which each depends. |  |
| b) | With a neat circuit diagram, explain the Swinburne’s test on d.c. machine. How efficiency of d.c. machine is calculated from it? |  |
| c) | Explain the procedural steps for performing Hopkinson’s test on d.c. machine with a neat circuit diagram. State its advantages and disadvantages. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the necessary tests to be performed on a transformer in order to know hysteresis losses & eddy current losses separately. Explain in brief how it is performed. |  |
| b) | Enlist various tests to be performed on a transformer under ‘Dielectric testing’. Explain any one in detail. |  |
| c) | Draw the circuit diagram with measuring instruments and their range for performing open circuit test on a 100KVA, 11KV/400V Δ/Y transformer. Also explain the procedure. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain the methods of measurement of temperature rise of various parts of induction machine. |  |
| b) | Explain with a neat diagram open circuit voltage ratio test on a slip ring induction motor. What is the significance of this test? |  |
| c) | A 6 pole, 50Hz, 3 induction motor running at full load develops a useful power of 160 N-m when the rotor emf makes 120 complete cycles per minute. Calculate the shaft output power. If the mechanical torque lost in friction, windage & core is 10 N-m, compute i) Rotor Cu loss ii) Input to the motor R iii) efficiency if the motor copper losses are 800W. |  |
|  |  | …….2/- |
|  | -2- |  |
| d) | List down the various type of tests to be performed on three phase induction motor. Explain No load test on three phase induction motor. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the maintenance schedule for induction machine as per I.S. |  |
| b) | How will you compute the efficiency of synchronous machine from losses as per IS4389-1968? |  |
| c) | What is the purpose of conducting sustained 3 short circuit test on a 3 synchronous machine? State its features. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Inspection of transformer on arrival at site |  |
| b) | Care of electrical equipment during the period of inactivity |  |
| c) | Commissioning test on 3 induction motors |  |
| d) | Calculation of parameters of equivalent circuit of transformer |  |

***********