## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Energy Conversion(ME405) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any six important applications of compressed air. |  |
| b) | State three important differences between 2-stroke and 4-stroke engines. |  |
| c) | State three advantages of MPFI engine over carburetted engine. |  |
| d) | State three important features of hydro-electric power plant. |  |
| e) | State any six applications of solar energy. |  |
| f) | State three important considerations in site selection of a wind power plant. |  |
| g) | State three important factors that determine the performance of a bio-gas plant. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of a centrifugal compressor. |  |
| b) | With the help of a neat sketch, explain the construction and working of a screw compressor. |  |
| c) | Draw p-v diagram of a single stage reciprocating compressor with clearance. Define volumetric efficiency of the compressor. State the factors on which volumetric efficiency depend. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A diesel engine has a compression ratio of 20 and cut-off takes place at 5% of the stroke. Find the air-standard efficiency. Assume γ=1.4. |  |
| b) | With the help of a neat sketch, explain the working of a 2-stroke cycle petrol engine. |  |
| c) | Draw a neat labelled sketch showing layout of a Thermal power plant and explain its working. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An engine uses 6.5Kg of oil per hour of calorific value 30,000 KJ/Kg. If the brake power of the engine is 22 KW and mechanical efficiency 85%, calculate, - Indicated thermal efficiency - Brake thermal efficiency - Specific fuel consumption in Kg/Brake Power/Hour. |  |
| b) | With the help of neat labelled sketch, explain the ignition system of a petrol engine. |  |
| c) | Draw a neat layout of a closed cycle gas turbine power plant and explain its working. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled sketch of a horizontal axis wind turbine and state its advantages and disadvantages. |  |
| b) | Draw a neat labelled sketch of a Bio-Gas plant and explain its working. | ……2/- |
|  | -2- |  |
| c) | Draw a neat labelled sketch of a solar still and explain its working. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | State four advantages of multistage compression over single stage compression across same pressure limits. |  |
| b) | Draw a valve timing diagram for a 4-stroke cycle petrol engine and explain. |  |
| c) | State any four reasons for engine overheating and state the remedies. |  |
| d) | Draw a neat labelled sketch of a solar cooker. |  |
| e) | Explain any one method of converting wave energy to electricity. |  |

***********