## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Structural Drawing(4341) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Bonjean curves |  |
| b) | Use of shell expansion plan |  |
| c) | Section Modulus |  |
| d) | Longitudinal framing |  |
| Q.No.2. | Sketch the midship section of an oil tanker showing all structural details. Label the following members: - Longitudinal Bulkhead ii) Longitudinal iii) Girder iv) Bilge Plate | (12) |
| Q.No.3. | Sketch a typical shell expansion plan. Also show how the numbering of plates is done. List the use of shell expansion plan. | (12) |
| Q.No.4. | Answer the following questions: |  |
| a) | Explain in details how section modulus of a ship section is obtained. | (8) |
| b) | How will you ensure that the ship constructed using particular material will be able to take the required loading? | (2) |
| c) | What is safety factor? | (2) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Briefly describe hogging and sagging of a ship in still water and also when the vessel is poised on a wave profile. | (8) |
| b) | Explain how buoyancy curve is obtained from a typical wave profile. | (4) |
| Q.No.6. | A uniform box shaped barge, 40m x 12m beam is divided into four cargo compartments each 10m long. The barge is loaded with 600t of iron ore shown as follows: No.01 hold -135t No.02 hold -165t No.03 hold -165t No.04 hold -135t The loaded barge floats in fresh water on an even keel at a draught of 1.75m. Construct the curve of shearing force and bending moment for this condition and also find the position and the value of maximum bending moment for the still water condition. | (12) |
| Q.No.7. | Cross–section of a rectangular shaped barge is given in fig.1. Calculate the section modulus at deck and keel. All plating are 10mm thick. | (12) |