## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg.

Subject: Illumination Engineering(5106)/(EE612) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With the help of neat diagram, explain general lighting scheme. |  |
| b) | Define: (i) LOR (ii) DLOR (iii) ULOR. |  |
| c) | State the expression for finding the illuminance at a point P directly below the centre of the round disc source of light. Also state each term used in the expression. |  |
| d) | Explain any three factors affecting coefficient of utilization. |  |
| e) | State the formula used for calculation of number of luminaires by using lumen or light flux method. State each term used in the formula. |  |
| f) | State the factors affecting the selection of mounting height of streetlight luminaires. |  |
| g) | Explain ‘Waste light factor’ in flood lighting calculations. |  |
| h) | State any three important features of corridor lighting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | (i) Define light. What is radiant efficiency of a luminous body? | (2) |
|  | (ii) What are the advantages of artificial lighting produced electrically? | (3) |
|  | (iii) State the range of wavelength of electromagnetic radiations which produces lighting effect. | (1) |
| b) | Explain what is colour rendering index for a lighting source & explain its significance. Arrange the following lamps in ascending order of colour rendition: (i) Sodium vapour lamp (ii) Incandescent lamp (iii) Fluorescent lamp (iv) CFL. |  |
| c) | What is meant by stroboscopic effect? Explain with the help of neat diagram, how this effect is eliminated in fluorescent tube lighting? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain inverse square law with the help of neat diagram. |  |
| b) | A fluorescent lamp is mounted above the front edge of a workbench. Both are 1.5 m long & the mounting height is 1m. The transverse intensity is 450 cd at all angles. Calculate the illumination level at the centre of the workbench. |  |
| c) | (i) Define (1) Luminous Flux (2) Lumens (3) Luminous Intensity (4) Candle Power. | (4) |
|  | (ii) State ‘Lambert Cosine Law’. | (2) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out all the steps for indoor lighting design & preparation of layout of luminaire. |  |
| b) | Explain the various factors affecting coefficient of utilization and space to mounting height ratio. | ……2/- |
|  | -2- |  |
| c) | A hall 30m long and 12m wide is to be illuminated and illumination required is 50m candles. Five types of lamps having lumen output, as given below are available. Watts; 100; 200; 300; 500; 1000. lumens; 1615; 3650; 4700; 9950; 21,500. Taking a depreciation factor of 1.3 and utilization factor of 0.5, calculate the number of lamps needed in each case to produce required illumination. Out of the above five types of lamps, select most suitable type and design a suitable scheme & make a sketch showing location of lamps. Given S/hm = 1.2 |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State & explain different types of street light luminaires. Also state one application of each type of luminaires. |  |
| b) | List the steps involved in the design of street lighting. Also state the factor affecting the choice of luminaires used in street lighting. |  |
| c) | With the help of neat diagram, explain what is (i) Carriage way (ii) Kerb (iii) Outreach (iv) Overhang, related to street lighting. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is flood lighting? Explain the classification of floodlight projectors according to their beam spread. |  |
| b) | The front of a building 50m x 16m is illuminated by sixteen 1000 Watt lamps arranged so that uniform illumination on the surface is obtained. Assuming a luminous efficiency of 17.4 lumens/watt and coefficient of utilization of 0.4, determine the illumination on the surface. |  |
| c) | (i) State three special features of sports lighting. (ii) State three important features of spot lighting. |  |

***********