## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Architectural Engg.

Subject: Building Construction-V(4532) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer in short any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain any three situations in which pile foundation are preferred. |  |
| b) | Enumerate any three types of most commonly used partition walls. |  |
| c) | Name any three widely used materials for false ceiling panelling. |  |
| d) | Give three advantages of using built in furniture. |  |
| e) | List three materials used in making the shutters of built in furniture. |  |
| f) | Mention any three types of materials used in making furniture in hospitals. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch and explain foundation any two types for high rise buildings. |  |
| b) | Draw a detailed sketch through a basement built in R.C.C. labeling all its parts with dimensions and explain. |  |
| c) | i) A compound wall is to be designed in such a way that the footing should not be spread in neighbours property beyond the boundary line. What kind of footing would you suggest? Draw its plan and section. ii) What kind of partition wall would you suggest for a music room? Give two reasons for the choice. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with sketch stud partition and braced partition. |  |
| b) | Give six advantages of using curtain wall in high rise buildings. |  |
| c) | Write short notes on: i) Wood Wool partition ii) Plaster slab partition. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of detailed sketch, explain the construction of timber false ceiling with decorative materials. |  |
| b) | Give three advantages and three disadvantages of using false ceiling in construction. |  |
| c) | What kind of false ceiling would you suggest for a cinema hall? Give five reasons for the choice. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A wardrobe space measuring length 5m, depth 0.6m, height 2.10m is to be designed for a master bedroom in a residential bunglow. Suggest suitable materials and draw typical section, elevation and any two details, explaining the design with dimension. |  |
| b) | A display counter is to be designed for a jewellery showroom in a space of size length 2m, depth 0.3m, height 2m. Suggest the appropriate materials and draw typical section, elevation and any two details with dimensions. | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Give four differences of a capsule lift over a normal lift. |  |
| b) | Sketch a machine room of an elevator and label all its components. |  |
| c) | Give any four advantages of using mechanical means of vertical access compared to staircase. |  |
| d) | Give any four factors considered for locating a mechanical means of vertical access. |  |

***********