## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg./A

Subject: Machine Drawing(ME301) [Revised]

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Show how the roughness is indicated for the following situations: - Surface to be obtained by any production method - Surface to be given a machining allowance of 3mm - Sampling length of 12mm |  |
| b) | Show the double line representation of following Pipe fittings: - Cross ii) Reducing Socket iii) Tee |  |
| c) | Draw the symbols for following: i) Square butt weld ii) Fillet weld iii) Spot weld |  |
| d) | Indicate the roughness grade symbols for the following grade numbers: i) N3 ii) N5 iii) N10. |  |
| e) | Draw the symbols for the following: - Flatness iii) Parallelism iii) Circularity |  |
| f) | Interprete the meaning of geometrical tolerances shown in figure 1. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Draw the conventional representation of the following: - Lead ii) Rubber iii) Glass iv) Steel |  |
|  | ### OR |  |
| a) | With the help of a suitable free hand sketch show the following types of sections: - Offset section ii) Revolved section iii) Local section |  |
| b) | Figure 2 shows single line pipe layout. Convert it to double line pipe layout. |  |
|  | ### OR |  |
| b) | Show the single line representation for the following pipe fittings: - Gate Valve ii) Cap iii) 45 Lateral iv) Union |  |
| Q.No.3. | Draw the neat proportionate free hand sketches of any two of the following: i) Gib and cotter joint ii) Flat belt pulley with arms iii) Muff coupling | 2 x 6 = 12 …..2/- |
|  | -2- |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | Figure 3 shows the details of Knuckle joint. Draw the following views of assembly: - Sectional front view - Top view |  |
| b) | Figure 4 shows the details of the protected type flange coupling. Draw the following views of assembly: - Half sectional front view - View from the right |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | Figure 5 shows the assembly of socket and spigot joint. Draw the detail drawing indicating front view and top view of each of the parts. |  |
| b) | Figure 6 shows the assembly of plummer block. Draw the detail drawing indicating front view & top view of each of the parts. |  |
| Q.No.6. | Redraw the views given in figure 7 by proportionate free hand sketch and show appropriate welding symbols. Plates marked as 1,2,3 and 4 are to be connected as mentioned below: | (12) |
|  | i) Piece 1 is connected to piece 2 by single J butt weld. |  |
|  | ii) Piece 2 is connected to piece 3 by all round fillet weld arc welding. |  |
|  | iii) Piece 1 is connected to piece 4 by square butt weld finished flush on top. |  |