## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Machine Drawing-II(4035) [Old]

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Interpret the meaning of geometrical tolerances of the parts shown in fig. No.1. |  |
| b) | Calculate the limit sizes for a hole designated as  25H7. Following data is given: - Diameter 25mm lies in the range from 18 to 30 mm - Unit tolerance i=0.001D+0.453D (microns) - IT7 = 16i |  |
| c) | Indicate the roughness value Ra in microns for the following roughness grade number: (i) N1 (ii) N4 (iii) N12 |  |
| d) | Indicate with symbols each of the following: i) Machining finish with shaping to a surface roughness value of 3.2 microns. ii) Machining allowance of 3 mm iii) Sampling length of 12 mm iv) Surface to be electroplated. |  |
| e) | Sketch the orthographic sectional front view of the following pipe fittings: (i) Elbow (ii) Cross. |  |
| f) | Draw neat free hand sketch and the symbol for the following: - Cross sectional view of Double ‘J’ butt joint - Cross sectional view of Double bevel butt joint |  |
| Q.No.2. | Draw neat proportionate free hand sketches of any two of the following: (i) Knuckle joint (full sectional front view) (ii) Unprotected flange coupling (full sectional front view) (iii) Step cone pulley (full sectional front view) | 2 x 6 = 12 |
| Q.No.3. | Fig. No.2 shows the assembly of a simple eccentric. Draw front view and top view of each part separately, name the parts and prepare the bill of materials. | (12) |
|  | ### OR |  |
| Q.No.3. | Fig. No.3 shows the details of the gland and stuffing box. Draw the following views of the assembly according to the third angle method of projection. - Half sectional front view - Top view | (12) |
| Q.No.4. | Fig. No.4 shows two orthographic views of the Jig plate of a drill jig. Redraw the views and incorporate the following details: (i)Bottom surface of the jig plate to be indicated as datum ‘A’ (ii) To face to be parallel to datum ‘A’ within a tolerance of 0.02mm (iii)Tolerance of g6 on thickness of 25 mm (iv)Tolerance of H7 on  54 mm (v)Roughness of the inner surface of hole of  54 mm to be indicated by . (vi)Radial run out of 0.1 mm on the cylindrical surface of  216 mm. | (12) ……2/- |
|  | -2- |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Fig. No.5 shows a composite welding of a plate and studs weld. Represent the following on the given views to join (i)Plate 1 to 2 – fillet weld all round (ii)1 to 3 - single – U butt weld on site (iii)1 to 4 – Double – U butt weld on site finished flush at the top (iv)3 to 5 – continuous fillet weld on the front side and intermittent fillet weld on the rear side starting with a weld length of 20 mm followed by a gap of 30 mm. |  |
| b) | Fig. No.6 shows a piping layout in the form of a block diagram with the names of various fittings mentioned in the blocks. Draw a single line diagram for the same. |  |
| Q.No.6. | Study the working drawing of a slip bush shown in fig. No.7 and answer any six of the following: | (12) |
|  | i) What is the size of the hole in the bush? |  |
|  | ii) What is the meaning of 3x45? |  |
|  | iii) What is the depth of the drilled hole at the side of the bush? |  |
|  | iv) What is the tolerance provided on hole of  16 mm? |  |
|  | v) What is the method used for imparting a rough surface to the bush for the purpose of gripping ? |  |
|  | vi) What is the roughness of the inner surface of the hole of  16 mm? |  |
|  | vii) What is the size of the bar stock required for the manufacture of the bush? |  |