## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Machine Design Practice(4045)/(ME601) [Old/Rev.]

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Discuss briefly the general steps followed in the design of Machine elements. |  |
| b) | State the applications and advantages of Knuckle joint. Explain the various types of failures in the Knuckle joint. |  |
| c) | What do you mean by fatigue strength? Draw S-N curve showing endurance limit. |  |
| d) | Find the minimum size of the hole that can be punched in a 20mm thick M.S. plate having an ultimate shear strength of 300 MPa. The maximum permissible compressive stress in the punch material is 1200 N/mm2. |  |
| e) | Sketch the various thread profiles used in Power screws. Why are square thread profiles preferred over V-thread profiles? |  |
| f) | What is Wahl’s stress correction factor? Why is it considered in the design of Helical springs? |  |
| g) | Explain the modes of failure of a rectangular sunk key. Write their expressions to obtain the dimension of the key. |  |
| h) | Draw stress-strain diagram for mild steel and explain Elastic limit, Yield stress & Ultimate tensile strength. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two rods made up of plain carbon steel 30C8 (yt=400N/mm2) are to be connected by means of socket and spigot type cotter joint. The diameter of each rod is 40mm. The thickness of the cotter is 12mm. Assume the following relationships: Yield strength in compression, yc =1.5 yt Yield strength in shear =0.5 yt The factor of safety is 5. Calculate i) Inside diameter of the socket ii) Outside diameter of socket, and iii) Diameter of socket Collar. |  |
|  |  | …….2/- |
|  | -2- |  |
| b) | Design a bearing cap for a plummer block having shaft diameter of 100mm and length 150mm. The cap is fastened by means of 4 bolts two on each side of the shaft. The cap has to withstand a load of 18KN. The distance between the centre line of the bolts is 180 mm. Find the thickness of the bearing cap and diameter of bolts and deflection of cap. Assume the bearing cap is made up of cast iron with permissible stresses t= 20 N/mm2 & Young’s modulus of elasticity is E = 1.1 x 105 N/mm2. Bolts are made up of steel with permissible stress t= 50N/mm2. |  |
| c) | Fig.1 shows a right angled Bell crank lever. The load to be lifted ‘W’ is 682N, acting on a pie at the end of 140mm arm and effort ‘P’ is applied at the end of 190mm arm. The permissible value of tensile stress for the material of the lever is 80MPa, allowable bearing pressure at the pins is 8N/mm2. Assume the ratio of length to diameter of fulcrum pin is 1.25 and ratio of width to thickness of lever arm is 3. Design the following: i) Fulcrum pin at ‘F’ ii) Lever cross section of section Y-Y 40mm from the fulcrum pin. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | An offset link subjected to a force of 25KN is shown in the fig.2. It is made up of grey cast iron FG300 (ut = 300 N/mm2) and the factor of safety is 3. Determine the dimensions of the cross section of the link. | (3) |
|  | ### OR |  |
| a) | A bracket made up of cast iron FG200 is subjected to a load of 5KN as shown in the fig.3. Determine the required thickness ‘t’ of the cross section, if the factor of safety is 4. | (3) |
| b) | The layout of a transmission shaft carrying two pulleys B and C and supported on bearing A and D is shown in fig.4. Power is supplied to the shaft by means of a vertical belt on pulley B, which is then transmitted to pulley C carrying horizontal belt. The maximum tension in belt on pulley B is 2.5 KN. The angle of wrap for both the pulleys is 180 and coefficient of friction is 0.24. The shaft is made up of plain carbon steel 30C8 (yt = 400 N/mm2) and the factor of safety is 3. Determine the diameter of shaft on strength basis. Refer figure 4. | (9) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A structural bracket shown in fig.5 is subjected to an eccentric force P of 10KN with an eccentricity of 500mm. The centre distance between bolts 1 and 2 is 200mm, and the centre distance between bolts 1 and 3 is 150mm. All the bolts are identical. The bolts are made up from plain carbon steel 30C8 (yt = 400 N/mm2) and the factor of safety is 2.5. Determine the size of the Bolts. | …….3/- |
|  | -3- |  |
| b) | i) Explain ‘Bolts of uniform strength’. Draw neat sketches of the same. ii) A plate 100mm wide and 10 mm thick is joined together with another plate by a single transverse and double parallel fillet weld as shown in fig.6. The permissible tensile stress and shear stress for plate and weld materials are 70 N/mm2 and 55 N/mm2 respectively. Find the length of each parallel fillet weld if the joint is subjected to static load. |  |
| c) | Design a cast iron protective type flange coupling to transmit 15 KW at 900 rpm, from an electric motor to a compressor. The following permissible stresses may be used: - Shear stress for shaft, bolt and key material = 40MPa. Crushing stress for bolt, and key = 80MPa Shear stress for cast iron = 8MPa. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Design a screw jack for lifting a load of 50KN through a height of 0.4m. The screw is made up of steel and nut of bronze. The following allowable stresses may be assumed For steel: Compressive stress = 80MPa; shear stress = 45MPa. For bronze: Tensile stress = 40MPa, Bearing stress = 15MPa and shear stress = 25MPa. The coefficient of friction between the steel and bronze pair is 0.12. The screw should have square threads. Obtain the dimensions of the screw and nut only. |  |
| b) | i) Explain the effect of keyways on strength of the shaft. ii) Give the step by step procedure involved in the design of muff coupling with the help of neat sketch of the coupling. | (2) (4) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a lead spring for the following specifications: i) Central load= 100KN ii) Distance between spring support = 1m iii) Central band width = 80mm, permissible shear stress = 300MPa. The ratio of depth to width of spring = 3. Determine i) Thickness & width of leaves ii) Deflection of spring. |  |
| b) | Design a close coiled helical spring for maximum load of 4.5KN and deflection is 40mm. The allowable shear stress is 500MPa and spring index C = 6. Take G= 0.8 x 105 MPa & K = 4C-1 + 0.615 4C-4 C |  |
| c) | i) Explain the following with respect to bearings: 1) Sliding Contact bearing 2) Roller Contact bearing. ii) What is meant by bearing life? Also explain basic static & dynamic load in bearings. | …..4/- |