## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Instrumentation-I(4214) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the block diagram of an instrumentation system. |  |
| b) | Briefly explain primary and secondary standards. |  |
| c) | Compare the performance of a wire wound potentiometer with other types of potentiometers. |  |
| d) | Give one example each of contact type and non-contact type of instrument. |  |
| e) | State Piezoelectric effect. |  |
| f) | Briefly describe an IC sensor used for temperature measurement. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit of null and deflection type instrument and compare the performance. |  |
| b) | Explain the objectives of an instrumentation system. |  |
| c) | Explain how direct, indirect and routine calibration is carried out. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe self generating and power operated instruments with the help of example. |  |
| b) | Explain the working of an electromagnetic transducer with the help of a diagram. |  |
| c) | Explain the principle of working of Bourdon tube and Flapper nozzle. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain one application of potentiometer as a transducer using block diagram. |  |
| b) | Explain why and how compensation techniques are used in strain gauge circuits. |  |
| c) | Explain the construction and working of a photoconductive cell. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare the performance of the three types of capacitive transducers. |  |
| b) | Describe the application of capacitive transducer for measurement of displacement with the help of block diagram. |  |
| c) | Explain how differential output arrangement for transducers is useful with help of diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of inductive transducer based on variation of self-inductance. |  |
| b) | Explain the application of inductive transducer in the measurement of pressure with the help of a block diagram. |  |
| c) | Explain construction and working of Thermistor. |  |

***********