## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Power Plant Engineering(5037)/(ME623) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following: i) Maximum demand ii) Average load iii) Load factor. |  |
| b) | Draw a neat sketch of open cycle gas turbine Power Plant. |  |
| c) | What are the different types of fuels used for steam generation? |  |
| d) | Why is feed water treatment necessary before supplying to the boiler? |  |
| e) | What factors are to be considered while selecting a site for Hydel Power Plant? |  |
| f) | What is the function of cooling tower in Power Plant? |  |
| g) | What is nuclear fission? Explain in brief. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A power station supplies the following loads to the consumers: |  |
| Time in hours; 0-6; 6-10; 10-12; 12-16; 16-20; 20-22; 22-24. Load in MW; 30; 70; 90; 60; 100; 80; 60. |
|  | i) Draw the load curve and estimate the load factor of the plant. ii) What is the load factor of a standby equipment of 30MW capacity if it takes up all loads above 70MW? |  |
| b) | What are advantages & disadvantages of hydro electric Power Plants over thermal Power Plants? |  |
| c) | Draw the layout of diesel engine Power Plant. What are the different fields where the use of I.C. engine Power Plant is essential? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How do we do the thermal refinement of simple open cycle gas turbine? Explain. |  |
| b) | With a neat sketch explain the general layout of a steam Power Plant. |  |
| c) | With a neat sketch explain the working of electrostatic precipitator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the working of cyclone burner. Why is this method more preferred over other methods of burning? |  |
| b) | List and explain the site selection criteria for thermal Power Plant. |  |
| c) | Explain the following water treatment methods: i) Ion- Exchange Process ii) De-aeration Process. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the induced draft type of a cooling tower. |  |
| b) | Explain the construction & working of Breeder reactor. | ……2/- |
|  | -2- |  |
| c) | Explain briefly the various methods of disposal of radio active waste. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Load curve & its significance in Power generation |  |
| b) | Lubrication system in diesel Power Plant |  |
| c) | Necessity of Ash handling system |  |
| d) | Functions of feed water loop |  |
| e) | Types of Hydel Power Plants |  |
| f) | Hyperbolic cooling tower |  |

***********