## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Architectural Engg.

Subject: Building Construction-II(AE206) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give an example of a composite building component in the superstructure having at least three building materials used into it. |  |
| b) | Define ‘bearing capacity of soil’. |  |
| c) | Give reason why English Bond is stronger than Flemish bond. |  |
| d) | Define ‘rise’ of an arch. |  |
| e) | Suggest a solution for preventing dampness in a timber flooring. |  |
| f) | List at least 4 selection criteria for choosing a roofing material. |  |
| g) | What is the purpose of a Newel post in a staircase? |  |
| h) | Define ‘Reinforced Brickwork Masonry’. |  |
| i) | Define ‘Landing’ of a staircase. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between sub-structure and superstructure. (Give three points each) |  |
| b) | Sketch the sectional detail of roof having 23 slope covered with Mangalore tiles resting on load bearing wall of a verandah. |  |
| c) | Draw a neat sketch of a three quarter turn staircase reaching a floor height of 4m and label all its components. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Elaborate on the essential requirements of a good foundation. |  |
| b) | Draw a neat labelled and dimensioned diagram in plan and section of a R.C.C. sloped footing for a framed structure. |  |
| c) | Explain the process of setting out of foundation wall trenches for a load bearing structure. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write at least six important points to be considered while supervising brick masonry. |  |
| b) | Sketch any three of the following types of joints in stone masonry: i) Rebated or lapped joint ii) Dowelled joint iii) Juggle joint iv) Bed joint |  |
| c) | Explain with sketches any two types of Brick-Stone composite masonry. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch and write a short note on any two of the following: - Stone lintel ii) wooden lintel iii) Brick lintel |  |
| b) | Explain the process of scaffolding work for the construction of an arch. | ……2/- |
|  | -2- |  |
| c) | Write short notes on the following components of a staircase: - Nosing ii) Stringer iii) Flight iv) Going |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the details of laying of i) asphalt flooring ii) PVC flooring. |  |
| b) | Write short notes on the following types of plaster finishes: i) Smooth cast finish ii) Sand faced finish |  |
| c) | Explain the fixing of any two wall cladding materials for a residential bungalow. |  |

***********