## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Computer Engg.

Subject: Microprocessors & Applications(CO401) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Discuss any two control flags in 8086. |  |
| b) | Explain the following 8086 instructions: i) CMP ii) AND iii) MUL. |  |
| c) | Compare Macros and Procedures. |  |
| d) | Explain the following pins/signals in 8086: i) DTRii) ALE iii) AD0−AD15. |  |
| e) | List any 3 features of Pentium processor. |  |
| f) | Discuss the role of instruction pointer in 8086. |  |
| g) | Explain any three assembler directives of 8086. |  |
| h) | Explain the different types of interrupts with respect to 8086. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the block diagram of 8086 microprocessor. Explain the role of Index registers in 8086. |  |
| b) | Explain with a neat diagram the flag register of 8086. |  |
| c) | With a neat diagram explain memory segmentation in 8086. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain string instructions in detail with suitable examples. |  |
| b) | Explain in detail the classification of memory addressing mode with necessary examples of each. |  |
| c) | Write an 8086 assembly language program to perform multiplication of two 16-bit hexadecimal values. Also explain processor control Instructions. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the various assembly language program development tools. |  |
| b) | Discuss the syntax of a Macro. Also explain how parameters can be passed to a macro. |  |
| c) | Explain how CALL and RET instructions are used to implement procedures in 8086. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat timing diagram, explain 8086 write machine cycle. |  |
| b) | Explain 8086 in minimum mode. Explain the functions of pins used in minimum mode. |  |
| c) | Draw the interrupt pointer table. Also discuss the concept of priority on Interrupts. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the Traffic Light Controller application. |  |
| b) | With a neat diagram explain how 8086 can be interfaced with Analog to Digital converter. |  |
| c) | List and explain the architectural features of Pentium processor. |  |

***********