## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engg.

Subject: Machine Tools & Production Process-I(4050) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between Hand tools and machine tools. |  |
| b) | Define i) Cutting speed ii) Depth of cut iii) Feed. |  |
| c) | What are the functions of Tailstock of a Lathe? |  |
| d) | Define machinability. List any two factors on which machinability will depend on. |  |
| e) | How will you determine Least Count of a micrometer Screw-guage? |  |
| f) | What are the special features of Power Hacksaw? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch show the geometry of Twist drill. |  |
| b) | Explain with neat sketch the use of V-Block while marking. |  |
| c) | Give the applications of the following: i) Ball peen Hammer ii) Cross peen Hammer iii) Soft Hammer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a Centre Lathe and label its various parts. |  |
| b) | What are Lathe Mandrels? List various types of mandrels and sketch any one and briefly explain it. |  |
| c) | What is positive Rake and Negative Rake angle of a cutting tool? Give its significance. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the major differences between a three jaw chuck and four jaw independent chuck. |  |
| b) | A shaft 1200mm long has a taper of 1:200 for a length of 600mm. The maximum diameter of the shaft is 75mm. Determine the minimum diameter of the shaft and the amount of setover required. |  |
| c) | Give the classification of single point lathe tool. Explain with neat sketch Brazed Tipped tool. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the various types of files and sketch any two. |  |
| b) | With neat sketches explain the following lathe operations: i) Plain Turning and shoulder Turning ii) Chamfering. |  |
| c) | Find the time required for one complete cut on a piece of work 350mm long and 50mm in diameter. The cutting speed is 35m/min and feed is 0.5mm/rev. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are different characteristics of the cutting tool material? |  |
| b) | List various different cutting tool materials. Explain in brief any two of them. |  |
| c) | State the various properties which cutting fluids should possess. |  |

***********