## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(4108)/(EE511) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define: i) Feeder ii) Distributor iii) Service mains. |  |
| b) | List the system voltage and permissible voltage drop in various parts of distribution system for urban, suburban & rural areas. |  |
| c) | List the types and sizes of conductors along with their corresponding current carrying capacity for rural electrification. |  |
| d) | State any three IE rules or standard practices followed for light and fan wiring. |  |
| e) | Define earthing. State the necessity of earthing. |  |
| f) | List any 6 factors affecting earth resistance. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State any six points of comparison between radial, ring and interconnected systems of distribution. |  |
| b) | Explain the SWER system of rural electrification. State its advantages and disadvantages. |  |
| c) | With a neat labelled diagram explain plate earthing. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State any explain any four factors on which selection of size of conductor for a distribution line depends. Also list the names and current capacity of conductors used for distribution purpose. (any four) |  |
| b) | A two wire radial AC distributor 1200m long supplies a load of 100A at 0.8 p.f. lag at the farthest end and a load of 60A at 0.9 p.f. lag at the mid point of the distributor. The power factor at the two load points are referred to the voltage at the farthest point. The impedance of each line is (0.15 + j0.2)Ω. If the voltage at the farthest point is maintained at 220V, calculate the voltage at the sending end. |  |
| c) | Find the inductance per Km of a 3, 3 wire transmission line having conductor of 1.24cm diameter and when the conductors are placed at:- - The corners of equilateral triangle of side = 2m - The corners of a triangle of sides 2m, 2.5m & 4.5m. |  |
| d) | State the types of faults. Briefly explain effect of each fault on the system. (Only for students of old scheme) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the main components of distribution line and state the function of any 8 components. |  |
| b) | Estimate the requirement of main material for erecting a 3, 4 wire, 11KV overhead distribution line over a straight run of 1Km length. Consider the pole span as 100m and horizontal conductor configuration. Use 11m rail poles and ACSR conductor 6/1 x 2.59 mm weighing approximately 120Kg/Km. Make any other suitable assumptions wherever required. | …..2/- |
|  | -2- |  |
| c) | A line is to be tapped off from 3, 11KV line to supply to a load at 3, 440V. The load is comprising of 3 pump motors and other machines totaling to 100 HP. Estimate the capacity (power & current) and other necessary specifications of transformer required. Assume that there may be 20% increase in load in the future. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare between cleat and conduit wiring based upon atleast 12 points/factors of comparison. |  |
| b) | With neat diagram briefly explain: - Staircase wiring & ii) Godown wiring |  |
| c) | A hall of 15m x 6m x 4.5m is to be wired up for providing 10 lamps of 100W each and 4 fans of 60W each. Draw the layout/plan showing the distribution/location of the lamps and fans. Also, calculate the number of subcircuits and ratings of wires and main switch required to be selected for their wiring work. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain: i) Cable system & ii) Overhead busway system, used for industrial purposes. |  |
| b) | A 10 HP, 415V, 3, 50Hz, 0.8 p.f. induction motor it to be installed in a workshop. Show the typical layout of wiring required for the same. Select the ratings of conduits, main switch, motor switch, fuses, cables and other relevant material required for the same. Make suitable assumptions wherever necessary. |  |
| c) | State important considerations/IE rules regarding motor installation wiring. |  |

***********