## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Electronic Circuits(4135) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw h-parameter equivalent of a transistor in CB configuration. |  |
| b) | How bandwidth of an amplifier can be determined from its frequency response for RC coupling? |  |
| c) | Explain the following with respect to power amplifier: (i) Conversion efficiency (ii) Collector dissipation. |  |
| d) | Calculate the frequency of oscillation of Colpitts oscillator given C1=0.001µF C2=0.02µF and L=15µH. |  |
| e) | Explain in brief the construction of UJT. |  |
| f) | Name the three types of multivibrators and differentiate between them. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the method of determining the ‘h’ parameters from input and output characteristics of CE amplifier. |  |
| b) | With the help of circuit diagram explain the working of direct coupled amplifier. State its applications. |  |
| c) | Explain the effect of coupling capacitor and emitter bypass capacitor on the low frequency response of an amplifier. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For a transistor configured in common base mode, calculate Zi, Zo, Av and AI. Given: Rg=600, RL=1.5K, hob=0.5x10-6 mhos, hib=20Ω, hrb=3x10-4, hfb = -.98 |  |
| b) | Explain high frequency response of CE amplifier. What is gain bandwidth product and short circuit current gain? |  |
| c) | Explain the need for power amplifiers. Explain in details harmonic distortion observed in power amplifiers. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain complementary symmetry push pull amplifier with help of circuit diagram. |  |
| b) | Explain the working of a Hartley’s oscillator with the help of circuit diagram. How is it different from Colpitts oscillator? |  |
| c) | Draw the block diagram of voltage series feedback amplifier and explain how feedback takes place. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how FET can be used as a voltage variable resistor. |  |
| b) | Explain the CS configuration operation of a FET with help of diagram. How is it different from CD configuration? |  |
| c) | With the help of circuit diagram explain the working of astable multivibrator. | …….2/- |

|  | -2- |  |
| --- | --- | --- |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With help of circuit diagram explain the working of Schmitt trigger. Show the graphical representation of hysteresis in a Schmitt trigger. |  |
| b) | Explain the working of emitter follower with the help of circuit diagram. State its applications. |  |
| c) | Explain the working of an SCR. Draw the characteristics and mark all important parameters on it. |  |

***********