## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Industrial Instrumentation(4218)/(EN607) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw sketch of two types of orifice plates and mention one feature of each. |  |
| b) | Compare characteristics of RTD and Thermocouple. |  |
| c) | Draw a diagram of pressure measurement system using LVDT and appropriate pressure transducer. |  |
| d) | State advantages and disadvantages of capacitive type level indicator. |  |
| e) | Discuss briefly the techniques used to measure flow in open channel. |  |
| f) | Describe application of I to P convertor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a magnetic flow meter with the help of a diagram. |  |
| b) | Explain the working of a Rotameter with the help of a diagram. |  |
| c) | Explain the working of TDT type of ultrasonic flow meter with the help of a diagram. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of working, construction and characteristics of RTD. |  |
| b) | Explain the working of a vapour pressure thermometer with the help of a diagram. |  |
| c) | Describe a non-contact method of measurement of temperature of very hot object with the help of a diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss various inherent errors in Bourdon tube and suggest suitable remedies in each case. |  |
| b) | Explain the construction and working of Pirani guage. State the range of Pirani gauge. |  |
| c) | Explain the construction and working of micro manometer. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain torque tube displacer method of level measurement with the help of a diagram. |  |
| b) | Explain then working of air purge system in level measurement with the help of a diagram. |  |
| c) | Explain the radiation method of level measurement with the help of a diagram. |  |
|  | (only for candidates of revised course) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the need for mass flow meter. Explain the principle of coriollis mass flow meter. |  |
| b) | Explain the working of a pressure to current convertor with the help of a diagram. | …….2/- |
|  | -2- |  |
| c) | Explain the working of a float type level measurement system with the help of a diagram. |  |
|  | (only for candidates of old course) |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain a practical application of a closed loop control system for temperature measurement with the help of a neat block diagram. |  |
| b) | Draw the block diagram of a level control system and explain the function of each block. |  |

***********