## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Architectural Engg.

Subject: Theory of Structures(AE405)/Structures-I(4537) [Old/Rev.]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between resolution and composition of forces. |  |
| b) | What is a couple? Enumerate its various characteristics. |  |
| c) | What is water cement ratio? State the importance. |  |
| d) | Enlist basic structural elements in brief and mention their behaviour. |  |
| e) | Explain in brief structural and non-structural cracks. |  |
| f) | Define bending moment and shear force. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A particle is acted upon by the system of forces shown in fig.2a. Find the magnitude and direction of the resultant. |  |
| b) | State and explain the principle of transmissibility. Also explain the conditions of equilibrium. |  |
| c) | Determine the magnitude and directions of the resultant of the following set of forces acting on a body. i) 250N inclined at 40 with East towards North ii) 200N towards North iii) 300N towards North West iv) 350N inclined at 50 with West towards South. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the BMD and SFD for a simply supported beam of span 8m carrying two point loads of intensities 60KN and 80KN acting at 2m and 4m from left support respectively. |  |
| b) | Draw BMD and SFD of a cantilever beam of span 3m subjected to UDL of 5 KN/m over entire span. Also find the value of maximum bending moment. |  |
| c) | A simply supported beam of span 6m is subjected to UDL of intensities 10KN/m over entire span. Also a point load of 50KN is acting at the center. Draw BMD and SFD. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the terms centroid and center of gravity. Also give the position of centroid of following sections: i) Rectangle ii) Triangle iii) semicircle. |  |
| b) | State the difference between load bearing structure and framed structure. Draw the cross-section of load bearing wall foundation. |  |
| c) | State difference between: i) Long column and short column ii) One-way slab and two way slab iii) Working stress method and limit state method. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail how the following factors influence crack formation in building: i) Elastic deformation ii) Foundation movements. | …….2/- |
|  | -2- |  |
| b) | Enlist different methods used to prevent crack formation. Explain any one method. |  |
| c) | Explain with sketch the stresses produced in beam due to bending moment and shear force. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Grades of concrete |  |
| b) | Seismic effects on buildings |  |
| c) | Loads acting on structure (in brief) |  |
| d) | Rigid frame type structure |  |