## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Shop Floor Practice(SH302) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe a four way tool post of a centre lathe. |  |
| b) | State difference between sleeve and socket used on a drilling machine. |  |
| c) | Draw a neat sketch of an end mill and state its uses. |  |
| d) | What are the different driving mechanisms used in power press? |  |
| e) | What is bending? State the factors involved in bending. |  |
| f) | Mention the following codes used in programming for CNC turning: i) Program start ii) Longitudinal Turning cycle iii) Hold iv) Rapid transverse v) Linear interpolation vi) Program end. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a block diagram of a centre lathe and label its main parts. State function of each part. |  |
| b) | Write short notes on the following lathe accessories: i) Carrier and catch plate ii) Mandrel. |  |
| c) | Explain the various boring tool mountings used on horizontal boring machine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled sketch of a parallel shank twist drill and label its parts and angle. What is the cutting edge angle of twist drill? |  |
| b) | Explain with a neat sketch the main parts and working of a pillar drilling machine. |  |
| c) | With a neat sketch explain the main parts of Horizontal boring machine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify and list shapers. State advantages of hydraulic type shaper. |  |
| b) | Describe any four types of milling cutters with neat sketches. |  |
| c) | Describe the construction and working of the following attachment with neat sketch used on milling machine: |  |
|  | i) Hand operated circular milling attachment ii) Slotting attachment |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give two points of difference between: i) Punching and blanking operation ii) Combination die and progressive die iii) Mechanical Press and Hydraulic Press |  |
| b) | List the various power press driving mechanisms and explain any two in detail. |  |
| c) | What is spring back in connection to bending? Draw neat sketches to show (i) ‘V’ bending (ii) edge bending and (iii) U –bending of a plate. | ……2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Cutting tool material |  |
| b) | Classification of NC Machines |  |
| c) | CNC Machines |  |
| d) | Machining of Keyway on a shaper |  |
| e) | Specification of Centre lathe |  |
| f) | Specification of Shaper and Horizontal boring Machine |  |

***********