## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics & Communication Engg.

Subject: Communication Systems-IV(Mobile) [5141] [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List various control functions of a switching system. |  |
| b) | Explain the following facilities provided by telephone exchange: i) Conference call ii) No dialing calls. |  |
| c) | Explain the concept of (i) Tone only paging (ii) Voice paging. |  |
| d) | Differentiate between conventional mobile telephone system and cellular mobile system. |  |
| e) | List the functions of MTSO. |  |
| f) | Explain the features and functions of Access channels. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain block diagram and operation of single processor SPC system. |  |
| b) | Explain load sharing mode of dual processor SPC configuration. |  |
| c) | Explain horizontal and vertical decomposition in distributed SPC. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of memory controlled time division space switching system with neat diagram. |  |
| b) | Explain the operations of random input random output time division time switch. |  |
| c) | Explain the concept of time multiplexed time switch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of paging receiver unit with neat block diagram. |  |
| b) | Explain the process of establishing a call in conventional mobile telephone system. |  |
| c) | Explain the functions of base station and mobile receiver in cellular system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain following process in a cellular system: - Monitoring (ii) Receiving calls. |  |
| b) | Explain how the mobile to landline call is routed in cellular system. |  |
| c) | Explain the functions of network switching subsystem of GSM with neat diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the features of cell site antennas. |  |
| b) | Explain the process of sending data to a pager device. |  |
| c) | Explain the limitations of conventional mobile telephone system. |  |

***********