## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mining Engg.

Subject: Mine Ventilation(4297)/(MN503) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write short notes on Methane layering. |  |
| b) | What are the limitations of natural ventilation pressure? |  |
| c) | Why turbulent flow is avoided in underground ventilation? |  |
| d) | Give the advantages of splitting. |  |
| e) | What are the considerations given while constructing an evasee? |  |
| f) | Mention the disadvantages of booster fan. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the objectives of mine ventilation? Mention the different sources of heat in mines. |  |
| b) | Explain the sources, physical, chemical & physiological properties of carbonmonoxide. |  |
| c) | Explain the principle & construction of flame safety lamp? What are the tests conducted by it? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with neat sketches i) Regulator ii) Aircrossing. |  |
| b) | Derive the Atkinson’s formula. In a drift of size 2.5m x 3.5m & length of 350m, 3000 m3/min of air travels. Calculate the w.g. expended in ventilating a drift if coefficient of resistance being 0.00161. |  |
| c) | Write a brief note on ‘Motive Column’. Air temperature in a downcast shaft 465m deep is 32C and that in upcast shaft of 250m depth is 38C. What assistance expressed in H.P, does this difference in temperature render when quantity of air passing through shaft in 400 m3/min? Assume density of air in DC shaft to be 1.2 Kg/m3. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of ‘Equivalent orifice’. Total quantity of air passing through a district of a mine is 30 m3/sec with a w.g. of 30mm. It is now proposed to reduce the quantity of air flowing through the district to 25 m3/sec. Calculate the size of the regulator that should be fitted in the district. |  |
| b) | Explain the features of ‘Reversible method’ of ventilating long heading with auxiliary fans supported by neat sketch. |  |
| c) | What are the objectives of ventilation survey? How is the pressure survey carried out in mines? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the principle & working of centrifugal fan. | …..2/- |
|  | -2- |  |
| b) | (i) Mention the different parts of an axial flow fan. What are the different ways of increasing the quantity delivered with regard to blade design? (ii) Calculate the w.g. produced by a 3m diameter radial centrifugal fan running at 30 rpm and delivering 6000 m3/min of air. Assume the density of air to be 1.2 Kg/m3. | (3) (3) |
| c) | Explain the different fan laws. A mine is ventilated by a fan producing 4000 m3/min at 75mm w.g. The fan runs at 300 rpm & absorbs 150 BHP. When the fan is speeded upto 400 rpm, calculate the new quantity, pressure & BHP. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a neat sketch of Coward’s diagram & explain the main features. |  |
| b) | Differentiate between Ascensional & Descensional ventilation. |  |
| c) | Three galleries of resistance ‘R’ each are connected in parallel. What is the equivalent resistance? |  |
| d) | Define characteristics curves. What information do you get from it? |  |

***********