## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics & Communication Engg.

Subject: Mobile Communication(EN620) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe the criteria for handoff. |  |
| b) | Distinguish between cellular telephone system, original conventional mobile telephone system and landline telephone system. |  |
| c) | Describe the TDMA technique used in cellular systems. |  |
| d) | Describe the methods used to improve spectrum utilization in a cellular system. |  |
| e) | Describe the reverse channels in a CDMA cellular system. |  |
| f) | Describe features of the GPRS network. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the roof-mounted and high-gain mobile handset antennas with the help of their radiation patterns. |  |
| b) | Describe the logon and monitoring processes in a cellular system. |  |
| c) | With the help of a block diagram, explain the process of call routing in a cellular system from a mobile phone to a landline phone. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a block diagram, describe the operation of a basic cellular system. |  |
| b) | Explain how authenticating and encryption are performed in a CDMA cellular system. |  |
| c) | List three differences between: - 2G and 2.5G network technology - CDMA 2000 and W-CDMA |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why is handoff needed in a cellular system? Describe (i) Mobile–assisted handoff (ii) Delayed handoff. |  |
| b) | What is frequency reuse? Describe any two frequency reuse schemes used in cellular telephony. |  |
| c) | Describe cochannel and adjacent channel interference. How are they reduced? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the architecture of a GSM network with the help of a block diagram. |  |
| b) | Explain the process of authentication and encryption in a GSM network. |  |
| c) | Describe the soft handoff process used in CDMA cellular systems. List two advantages and two disadvantages of soft handoff. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Describe the setup channels used in a cellular system. | …..2/- |
|  | -2- |  |
| b) | Write a note on cell-splitting technique used in cellular systems. |  |
| c) | Write a note on EDGE. |  |
| d) | Describe the cell structure in cellular telephony and list the factors on which the service quality of a cellular system depends. |  |

***********