## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Principles of Air Conditioning(5047) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric charts and steam tables is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following psychrometric properties i) Wet bulb temperature ii) Absolute humidity iii) Degree of saturation. |  |
| b) | List the sources of heat that are considered for calculating system heat gain. |  |
| c) | What are the factors considered for selection of Air-filters in air conditioning? |  |
| d) | What are the advantages of Rectangular ducts as compared with circular ducts? |  |
| e) | What is Economic thickness of Insulation? |  |
| f) | List at least six desirable properties of an Ideal insulating material. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction and working of Aspirating Psychrometer. |  |
| b) | A sample of moist air is at 25C DBT and contains 10 grams of water vapour per Kg of dry air under a total pressure of 1.013 bar. Determine the following psychrometric properties of the air using steam table and empirical formulae: i) Dew point temperature ii) Relative humidity iii) Specific enthalpy. |  |
| c) | Main duct of an air handling unit carries 540m3 of air per minute. If the velocity of the air is limited to 300 m/min in the duct, determine the diameter of the duct. If this duct is to be replaced by a Rectangular duct with an aspect ratio of 1.2, determine the size of rectangular duct assuming the same velocity in both the ducts. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following Psychrometric processes and represent the same on Psychrometric chart: i) Cooling and humidifying ii) heating and dehumidifying. |  |
| b) | With the help of neat sketch explain ‘Summer Air conditioning System’ for hot and humid condition. Represent the process on Psychrometric chart. |  |
| c) | 500m3/min of air at 40C DBT and 20% RH is passed over an Air washer. If the air leaves the Air-washer at 25C DBT and 60% RH, determine the following: i) Amount of moisture added in Kg/hr ii) Efficiency of humidifier (Air- washer). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare Central Air conditioning system with Unitary Air conditioning system. |  |
| b) | Discuss the special features of Air conditioning system used in hospitals. -2- | …..2/- |
| c) | With a neat sketch explain the construction and working of ‘Window Air Conditioner’. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how the following heat sources are accounted for calculation of internal heat gain: i) Occupancy load ii) Lighting load iii) Appliance load. |  |
| b) | An office is to be Air conditioned with the following data known. Inside design condition: 25C DBT, 50% RH Outdoor design condition: 43C DBT, 27C WBT Room Sensible Heat gain: 5600 Watts Room Latent heat gain: 1400 Watts Bypass factor: Zero If the Return air is mixed with outside air before entering the cooling & dehumidifying coil in the ratio of 4:1 by weight, determine the following i) RSHF ii) ADP (Apparatus dew point) iii) Condition of air entering the coil iv) Capacity of the cooling coil. |  |
| c) | With a neat sketch explain the working and construction of Impact type humidifier. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the various methods of air cleaning in air conditioning equipments. |  |
| b) | Explain two position electrical control system. |  |
| c) | With a neat sketch explain the construction and working of Humidostat. |  |

***********