## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./EC/EI/COMP/E/E&EE/MED

### Subject: Digital Electronics-I(4134) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Convert 10100.100(2) to i) Octal ii) Hex iii) BCD. |  |
| b) | Write the truth table of 2–input OR gate & draw OR gate using following gates: i) NAND ii) NOR. |  |
| c) | Convert 1100101(2) to gray code and also draw the logic diagram for the same. |  |
| d) | Define following terms with reference to Flip Flops. i) Propagation delay ii) Hold time iii) Delay time. |  |
| e) | Define synchronous and asynchronous counter & state one advantage of synchronous over asynchronous counter. |  |
| f) | State and define any three characteristics of digital ICS. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Simplify following boolean expression using K-map and draw logical diagram for simplified expression. Y(A, B, C) = m (0, 2, 4, 6) |  |
| b) | Prove following boolean expressions using laws of boolean algebra: i) A+ĀB=A+B ii) AB+ĀC+BC=AB+ĀC. |  |
| c) | Perform following 8-bit arithmetic using 2’s complement method: i)-25(10) +15(10) ii)32(10)-16(10) |  |
| d) | Draw EX-OR gate using NAND gate and prove the output equation. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw S-R Flip Flop using NOR. Write and explain the truth table. |  |
| b) | Explain preset and clear signals with reference to S-R Flip Flop with neat diagram. Why are these signals called as asynchronous signals? |  |
| c) | Draw J-K Flip Flop using NAND. Write and explain the truth table. |  |
| d) | Explain edge triggering with reference to Flip Flop. Draw edge triggered D Flip Flop. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What is register? What are different types of registers? Draw diagram for any one of them. |  |
| b) | What is counter? Draw diagram of 3-bit binary ripple counter. Write down the binary and equivalent decimal count. |  |
| c) | Draw diagram of 4-bit ring counter. Explain with timing diagram. |  |
| d) | Draw the counter which counts following count. B A 0 1 1 0 0 0 Also draw timing diagram for above counter. | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw diagram of 2-input RTL gate. Write and explain the same with truth table. |  |
| b) | Draw and explain CMOS inverter. |  |
| c) | Draw 2-input TTL logic. Explain truth table in brief. |  |
| d) | Define following TTL logic levels: i) VIH ii) VIL iii) VOH iv) VOL. |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Distinguish between TTL and ECL logic. |  |
| b) | Draw decade counter using J-K Flip Flop and write down the count. |  |
| c) | Write a shirt note on T Flip Flop. |  |
| d) | State and write Boolean expression for De Morgan’s laws. |  |
| e) | Write short note on ‘Racing condition’. |  |

***********