## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Chemistry of Welding Processes(4231) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer briefly: | 10x1.5= 15 |
| --- | --- | --- |
| a) | What are neutral oxides? Give an example. |  |
| b) | Why should a fuel have high calorific value? |  |
| c) | What is meant by deflagration & detonation in combustion? |  |
| d) | State any two properties of helium gas. |  |
| e) | Give an example of basic oxide. |  |
| f) | Define flux. |  |
| g) | What are corrosive soldering fluxes? |  |
| h) | State two advantages of liquid oxygen. |  |
| i) | Name any two shielding gases used in welds. |  |
| j) | What is the difference between soldering and brazing? |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | How is oxygen produced by electrolysis of water? |  |
| b) | What is the significance of oxidation reduction in welding? |  |
| c) | Compare helium and argon w.r.t. i) ionization ii) safety of its use and storage. |  |
| d) | State two properties and two uses of Calcium carbide. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write a note on explosibility of acetylene in mixture with other gases. |  |
| b) | How is gas quantity in a cylinder determined? |  |
| c) | What is the importance of LPG in welding? |  |
| d) | What are the properties and uses of an oxidising oxy-acetylene flame? |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write a note on distribution of specific heat flux. |  |
| b) | Why is it necessary to use a good flux in welding? |  |
| c) | State two functions performed by a brazing flux and two functions performed by a soldering flux. |  |
| d) | What are the disadvantages of using bare wire electrodes? |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4= 12 |
| a) | What are the functions performed by the following flux ingredients i) slag forming ingredients ii) gas shielding ingredients. |  |
| b) | Give a brief description of manufacture of electrodes by dipping method. |  |
| c) | What are the effects of the following gases absorbed in weld pool: i) hydrogen ii) oxygen. |  |
| d) | Explain briefly the following typical faults caused by gas absorption in welds: i) porosity ii) piping iii) blow holes. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write a note on ‘coated metals’ & ‘clad metals’. |  |
| b) | Give description of FCC in crystal lattices. |  |
| c) | What is the effect of temperature on crystal size in metals? |  |
| d) | State any four characteristics to distinguish between metals & non–metals. |  |

***********