## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(5036) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat sketch showing the power flow from the engine to the wheels in a 4WD vehicle. |  |
| b) | What is the function of receiver in the car air conditioning system? |  |
| c) | State one possible cause of ‘gear slipping’ and necessary remedy for the same. |  |
| d) | State the purpose of a torque wrench. |  |
| e) | Explain how tractive effort varies with vehicle speed in any particular gear. |  |
| f) | State the principle of operation of a hydraulic shock absorber. |  |
| g) | Define castor angle and state the reason why it is provided. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain monocoque construction of automobile chassis. |  |
| b) | With the help of a neat sketch, explain the construction and working of McPherson strut type of independent suspension. |  |
| c) | With the help of a neat sketch, explain linkage type of hydraulic power steering. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the advantages of MPFI engine over carburetion engine. |  |
| b) | List the various sensors used in an MPFI engine and state the location and function of each of them. |  |
| c) | With the help of a neat sketch, explain the fuel system of an MPFI engine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of a synchromesh gear box. |  |
| b) | With the help of a schematic diagram explain the principle of working of a simple automatic transmission. |  |
| c) | List the advantages and disadvantages of a fluid flywheel in an automobile. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of a lead-acid battery. |  |
| b) | Draw lighting circuit diagram to show the different lights used on an automobile. |  |
| c) | Explain the function of a starting drive. List the different types of starting drives. Draw a neat labelled diagram of any one type of Bendix drive. | …..2/- |

|  | -2- |  |
| --- | --- | --- |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Explain how the refrigerant compressor is driven. ii) With the help of a neat sketch, explain the function of thermostatic cycling switch in car air conditioning. |  |
| b) | State the provisions under section 56 of Motor vehicle Act 1988. |  |
| c) | With the help of a neat sketch, explain the working of any one mechanical or hydraulic jack. |  |

***********