## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

### Programme: Civil Engg./CT/FT&EE

Subject: Design of Structures-I(Steel) [4024] [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS-800-1984 Code and Steel Tables is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | State the different types of welds and the advantages and disadvantages of welded joint. |  |
| b) | Explain the different types of loads considered in the design of roof trusses and the load combination used. |  |
| c) | With the aid of neat sketch state the situations the plate girders are used and its various components stating their use. |  |
| d) | State the advantages and disadvantages of tubular sections. |  |
| e) | Explain the net area calculations for Angles and Tees used as Tension members. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A tie plate of 80x8mm is connected to gusset plate of 10mm thick subjected to tension. Permissible stress for tension in plate and shear in weld respectively are 150MPa and 108MPa. Design a fillet welded lap joint. |  |
| b) | Describe the procedure involved in the design of eccentric welded joint to connect bracket plate to column subjected to moment perpendicular to plane of welds. |  |
| c) | Design a fillet weld for connecting two angle section 2ISA 80x80x8mm on either side of gusset plate of 10mm thick subjected to a load of 165 KN. Take shear stress in weld as 110MPa. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | A tie member used is ISMC 300 @ 351 N/m connected to gusset plate by fillet welds. Design the fillet welds for the strength of tie member if overlap length of weld is limited to 350mm. If needed use slot welds. Take shear stress in weld as 108MPa. OR | (8) |
| a) | Design a suitable unequal angle section to carry a pull of 200 KN placed on the same side of gusset plate with tack welds. Also design the fillet weld connection for two angles adopted. Take shear stress as 108MPa. | (8) |
| b) | A tie member used is ISLC 100 @77.8 N/m connected to gusset plate by fillet welds. Determine the tensile strength of member. | (4) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Design a steel column adopting two channel sections with suitable spacing back to back to carry an axial load of 1000KN. The length of column is 7m with one end effectively restrained in position and direction and other in position only. OR | (8) …..2/- |
|  | -2- |  |
| a) | Design a slab base for a column ISHB 250 @ 547 N/m to carry a load of 800KN. Take bearing pressure of concrete as 4 N/mm2 and SBC of soil as 200 KN/m2. Also design pedestal. Show the sketch of connection of slab base with column. | (8) |
| b) | Differentiate between slab base and gusseted base. | (4) |
| Q.No.5. | Answer the following Questions: |  |
| a) | A simply supported beam of span 6m carry a UDL of 30 KN/m along with a point load of 40KN at centre of beam. The compression flange of beam is laterally supported. Designs a suitable section and check the section for shear and deflection. OR | (8) |
| a) | Design a floor beam to carry an UDL of 42 KN/m over an effective span of 8m. The depth of beam is restricted to maximum of 425mm. If necessary, flange plates may be used. Assume compression flange is laterally restrained. Carry out necessary check for shear, deflection and bending. | (8) |
| b) | A simply supported beam used is ISLB 250 @ 273.6 N/m for effective span of 5m. Determine safe UDL carried by the beam. | (4) |
| Q.No.6. | Answer the following Questions: |  |
| a) | The trusses for a factory building are spaced at 4.5m centre to centre. Purlins supporting roof coverings are provided over trusses at centre to centre spacing of 1.65m. Take vertical load from roof coverings as 165 N/m2 and slope is 28°, wind load normal to roof is 1100 N/m2. Design Angle purlin. OR | (8) |
| a) | Design a channel section purlin for a roof truss with spacing of trusses as 5m c/c and spacing of purlins as 1.6m c/c. Weight of AC sheets is 170 N/m2 and imposed load as 450 N/m2. Slope of truss 26 and wind pressure normal to roof as 1200 N/m2. | (8) |
| b) | Give stepwise procedure for calculating wind loads in sloping roofs. | (4) |

***********