## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric charts, Steam Tables and Mollier’s

charts is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe briefly what is geothermal energy. |  |
| b) | Explain the higher calorific value and lower calorific value of fuel. |  |
| c) | State i) First law of Thermodynamics ii) Second law of Thermodynamics. |  |
| d) | What do you understand by i) Sensible heat ii) Latent heat. |  |
| e) | Explain briefly the entropy of steam. |  |
| f) | Write down any three advantages of using an air pre-heater in boilers. |  |
| g) | What are the conditions for maximum efficiency in case of a two-stage reciprocating compressor? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the working of an Orsat apparatus for flue gas analysis. |  |
| b) | Represent and explain the following processes on P-V and T-S diagram: i) Isochoric process ii) Isobaric process. |  |
| c) | 0.2 Kg of air at a pressure of 105KN/m2 and temperature 15C is compressed adiabatically to a pressure of 2835 KN/m2. Calculate final temperature of gas and change in internal energy of the gas during compression. Take γ = 1.41 and Cv = 0.703 KJ/KgK. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with help of a neat sketch the working of any one type of steam calorimeter. |  |
| b) | Calculate the internal energy per Kg of superheated steam at a pressure of 10 bar and a temperature of 300C. If the steam is expanded to 1.4 bar and dryness fraction is 0.8, find the change of internal energy. Assume specific heat of superheated steam as 2.1 KJ/KgK. |  |
| c) | Explain with help of P-V and T-S diagram, the sequence of operations in the modified Rankine cycle. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain briefly with a neat sketch the working and construction of a Benson boiler. |  |
| b) | Explain briefly with a neat sketch the working of a Superheater. |  |
| c) | What is the objective of draught in a boiler? Explain briefly the different types of boiler draughts. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steam power plant is supplied with dry saturated steam at a pressure of 12 bar and exhausts into a condenser at 0.1 bar. Calculate the Rankine efficiency. | ……..2/- |
|  | -2- |  |
| b) | What is the function of a safety valve used in a boiler? Explain briefly with a neat sketch any one type. |  |
| c) | Explain the construction and working of a centrifugal compressor with help of a neat sketch. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Biogas energy |  |
| b) | Specific Heats of Gas |  |
| c) | Vane type Compressor |  |
| d) | Compressed Air Motor |  |

***********