## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Manufacturing Process-II(4036) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between orthogonal and oblique cutting. |  |
| b) | Name different types of lathes. |  |
| c) | What are the various methods of holding drill in drilling machine spindle? |  |
| d) | Differentiate between power press and fly wheel press. |  |
| e) | Define grit, grade and structure of a grinding wheel. |  |
| f) | What is centreless grinding? |  |
| g) | What is a progressive die? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How is taper specified? Discuss taper turning by swivelling the compound slide of a centre lathe. |  |
| b) | With a neat sketch explain the main parts of a fly press. |  |
| c) | Explain any three operations carried out on a drilling machine with sketches. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different characteristics of a good cutting tool material. List different cutting tool materials. |  |
| b) | Draw a neat labelled sketch of carriage of centre lathe and explain the uses of each part. |  |
| c) | List the various methods of holding work piece on a centre lathe. Explain any one with sketches. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by the following terms used with reference to grinding operation: - Loading and Glazing of grinding wheel - Dressing and truing of grinding wheel |  |
| b) | Differentiate between shaper and slotter. |  |
| c) | Describe any four types of cutters used in milling and their applications. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the salient features of a CNC lathe. |  |
| b) | Explain the following finishing operations: - Galvanising (ii) Electroplating. |  |
| c) | Explain the various types of work holding devices used on shaper. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Properties of cutting fluid |  |
| b) | Types of dies for press work |  |
| c) | Indexing attachment |  |
| d) | Power press drive mechanism |  |
| e) | Column and knee type horizontal milling machine |  |

***********