## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT

Subject: Civil Engineering Drawing(CE403) [Revised]

# Time Duration: 4 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Plan and draw a residential building with following details: 1. Area of plot: 340sq.m., 17m wide and 20m deep 2. Minimum set back: 3m on all sides 3. On G.F.: a living, a kitchen, 2 bedrooms, a bath, a W.C. and an attached wash room 4. A staircase of doglegged type 0.9m wide and tread of 0.3m 5. On F.F.: Just a living room & remaining all open terrace for future expansion 6. All walls 20 cm thick. Only wall between W.C. & Bath can be 12 cm thick |  |
| --- | --- | --- |
|  | i) Draw neat plan of G.F. to a scale of 1:50, following the basic planning principles, bye-laws & suitable dimensions for each unit. | (12) |
|  | ii) Draw plan of First floor including open terrace. | (6) |
|  | iii) Draw a section passing through stairs & toilets. | (8) |
|  | iv) Prepare an area statement including area of plot, area of G.F., area of F.F., total built up area permissible coverage, proposed coverage, permissible F.A.R & proposed F.A.R. | (6) |
|  | v) Draw site plan to a scale 1:500 including all necessary details. | (8) |
| Q.No.2. | Plan and draw any two of the following as line plan, to suitable scale: | 7.5x2=15 |
|  | i) A panchayat office with a function hall |  |
|  | ii) A primary school with at least 5 classrooms for about 200 children |  |
|  | iii) A Post office and a Bank extension in a single building Each line plan should include dimensions, passages, corridors, entrance steps, doors of suitable type & size, all tabulated neatly. |  |
| Q.No.3. | Answer any four of the following Questions: | 4 x 5 = 20 |
| a) | Differentiate between duties of an Engineer, Architect and Contractor involved in a construction project. |  |
| b) | Explain the building bye-laws in India/Goa in respect of open spaces, F.A.R, room dimensions and openings. |  |
| c) | Explain aspect, prospect, roominess, privacy and flexibility in planning. |  |
| d) | What are the details to be included in a site plan of a proposed building while submitting to the authorities? Make a list. |  |
| e) | Draw symbolic sketches to represent an European W.C., Towel rail, bathtub, washbasin and an open chowk (opening in plan) in a building. |  |

***********