## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Diesel Engine Operation & Maintenance(5045) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between trunk-piston engine and crosshead type engine. |  |
| b) | Define viscosity and show its significance as an important property of lubricating oil. |  |
| c) | What is ‘scavenging’ in diesel engines? |  |
| d) | Explain sludge formation in the lubricating oil of a diesel engine. |  |
| e) | Discuss the importance of preventive maintenance as against breakdown maintenance. |  |
| f) | Explain the use of piston ring compressor in diesel engine maintenance. |  |
| g) | State the inspection checks carried out on a cylinder liner during engine overhaul and name the measuring tools used for the same. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketches, explain the cycle of operations in a four-stroke diesel engine. |  |
| b) | List the methods of piston cooling. Explain any one with a neat sketch. |  |
| c) | List the various starting systems for starting diesel engines. With the help of a neat sketch, explain the compressed air admission starting system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the pre-starting checks to be carried out before starting a new or reconditioned engine. |  |
| b) | List the step-by-step procedure of adjusting the fuel injection timing of a diesel engine with jerk-pump fuel system. |  |
| c) | With the help of a neat sketch, explain the working of an idling and maximum speed type mechanical governor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List 6 important preventive maintenance checks for lubrication system of a diesel engine. |  |
| b) | Prepare a trouble shooting chart for the following complaint/ fault: High lubricating oil consumption. |  |
| c) | With the help of neat sketches, describe the various types of torque wrenches. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure of valve lapping, assembling the valves in the cylinder head and testing the valves for leakage. |  |
| b) | Explain the procedure of assembling the crankshaft on to the engine block. How is the crankshaft endplay adjusted? | …..2/- |
|  | -2- |  |
| c) | Explain how piston rings are inspected for wear and when decision is taken to replace them. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Supercharging |  |
| b) | Crankcase explosion |  |
| c) | Predictive maintenance |  |
| d) | Line boring |  |

***********