## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction to Thermodynamics(4334)/(SH404) Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam Tables is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Classify thermodynamic systems. |  |
| b) | Draw P-V and T-S diagram for Isentropic expansion process. |  |
| c) | Define ‘Compression Ratio’ for an Internal combustion engine. What are the usual compression ratio for Diesel engines? |  |
| d) | Define i) Enthalpy of evaporation ii) Enthalpy of superheated steam. |  |
| e) | Define i) Thermal conductivity ii) Emissivity. |  |
| f) | Differentiate between Dry compression and Wet compression. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain second law of thermodynamics with reference to Kelvin-plank and Clausius statements. |  |
| b) | i) A quantity of gas at 273K occupies 0.8 m3. What would be its volume at 327C, if the pressure is same at both temperatures. ii) A gas at a temperature of 27C and pressure of 1.5 bar occupies a volume of 0.105m3. If the gas is compressed to a pressure of 4.5 bar and volume of 0.04m3, what would be the final temperature of the gas? |  |
| c) | What are the functions of cooling system in an Internal combustion engine? Distinguish between water cooled and air cooled Internal combustion engines. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two kilograms of air at 700 KPa pressure and having a volume of 0.3m3 is expanded to a volume of 1.5m3. If the expansion takes place at constant pressure, determine the following: i) Final temperature ii) Work done iii) Heat absorbed or rejected by the air during the process. Assume R= 0.287 KJ/KgK and Cv = 0.711 KJ/KgK for air. |  |
| b) | 0.4 Kg of air at a pressure of 1 bar and temperature of 15C is compressed Isothermally to 5 bar pressure. Calculate the final volume and heat rejected during the compression process. Take R= 0.287 KJ/Kg-K for air. |  |
| c) | A quantity of oxygen occupies 0.9m3 at 5C and 147 KPa. It is compressed according to law PV1.25 = constant until the pressure is 2060 KPa. Determine i) the final volume ii) the final temperature iii) work done. Take R= 0.26 KJ/Kg K for oxygen |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat P-V diagram, explain Otto cycle. |  |
| b) | Explain with the help of neat sketch a Four-stroke cycle Diesel Engine. |  |
|  |  | ……2/- |
|  | -2- |  |
| c) | The following results were obtained from a test on a single cylinder oil engine. Brake specific fuel consumption : 0.31 Kg/KW/hr Calorific value of fuel oil : 43,960 KJ/Kg Brake Power : 26.1 KW Indicated power : 32.68 KW Calculate i) the mechanical efficiency ii) the Indicated thermal efficiency iii) the brake thermal efficiency of the engine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Use the steam table to find the enthalpy and specific volume per kg of steam of dryness fraction 0.85 at a pressure of 12 bar. |  |
| b) | How much heat is needed to convert ten Kg of feed water at 20C into dry saturated steam at a pressure of 9 bar? Take specific heat of water as 4.187 KJ/KgK. |  |
| c) | The glass window of a room has a total area of 12m2 and the glass is 5mm thick. The inside surfaces of windows are at 30C and the outside surfaces are at 10C. Calculate the quantity of heat that escapes from the room by conduction per second. The thermal conductivity of glass ‘K’ is 0.84 W/mK. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the principle of operation and working of vapour compression refrigeration system. |  |
| b) | Write a note on harmful effects of refrigerants on environment. |  |
| c) | A 10TR refrigeration system with R-134a as refrigerant works between -12C and 24C. If the refrigerant enters the compressor as dry saturated vapour and leaves the compressor at a temperature of 34C under superheated condition and leaves the condenser in saturated liquid state, determine the following: - Mass flow rate of refrigerant in Kg/sec - Quantity of heat rejected at the condenser in KJ/sec. - Theoretical C.O.P Use the following properties of R-134a Temp C; Enthalpy KJ/Kg. Liquid hf; Latent hfg; Vapour hg. -12; 184.16; 207.39; 391.55. 24; 233.05; 178.88; 411.93. Specific heat at constant pressure for vapour refrigerant = 1.018 KJ/KgK. |  |

***********