## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Engineering & Technology

Subject: Engineering Mechanics(CS301) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the characteristics of force for complete specification? |  |
| b) | Explain what is moment of a force and couple with example. |  |
| c) | What are the conditions of equilibrium for non-concurrent force system and parallel force system? |  |
| d) | State the Coulomb’s law of static friction. |  |
| e) | Explain law of conservation of momentum. |  |
| f) | What are the assumptions in analysis of trusses using method of joints? |  |
| g) | Explain what is equilibrant with example. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the resultant in magnitude and direction of a concurrent force system: i) 3KN acting at 20 ii) 4KN acting at 120 iii) 5KN acting at 270 iv) 6KN acting at 340. All the angles are measured with respect to positive X-axis. |  |
| b) | Find the resultant in magnitude, direction and its position from A. Refer fig.1. |  |
| c) | Find the resultant of the following parallel system and its position from A. Refer fig.2. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Using Lami’s theorem find the forces in the member AB and BC of jib crane lifting a load of 5KN at B. Refer fig.3. |  |
|  | OR |  |
| a) | Find the reaction at A and B of simply supported beam. Refer fig.4. |  |
| b) | Find the forces in the members of truss using method of joints. Refer fig.5. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A block resting on a rough horizontal plane required a pull of 9KN at 30 with horizontal just to move it. It was found that a push of 11KN inclined at 30 to the plane just moved the block. Determine the value of W and coefficient of friction. |  |
| b) | A ladder 3m in length of weight 200N is placed against wall making angle of 60 with the floor. The coefficient of friction between wall and ladder is 0.25, the coefficient of friction between floor and ladder is 0.35 and the ladder supports a man of weight 100N at its top. Calculate horizontal force P to be applied to the ladder at the floor lever to prevent slipping. |  |
| c) | A bullet weighting 0.25N is fired with velocity of 760m/s from a gun weighing 50N. Calculate the recoil velocity of the gun. If the length of the barrel of the gun is 1.00m, find the impulse and the impulsive force on the bullet. | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A man of mass 90 Kg is standing on the platform of stationary lift. What will be his weight if the lift is i) moving up with an acceleration of 1.8m/sec2 ii) moving down with an acceleration of 2.5m/sec2. |  |
| b) | Two masses 50Kg and 90Kg are connected by a thin weightless string passing over a light frictionless pulley. Find the tension in the string and acceleration of the system. |  |
| c) | A constant force 50N is acting on a block having mass of 16 Kg from t=0 sec to t=5 secs. The force suddenly increases thereafter to 75N. If at t=0 the body starts at rest, find the distance travelled in 10 seconds. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A spring has a length of 300mm when unstretched. If the stiffness factor is 30N per mm of spring, find the work done in stretching it to a length of 350mm. How much work is done in stretching the last 10mm (from 340mm to 350mm)? |  |
| b) | A water tank of 30 x 103 litres capacity is to be filled in 45 minutes by a pump, water being lifted through an average height of 15m. If the efficiency of the pump is 60%, find the rated power of the pump. |  |
| c) | i) A truck of mass 8 tonnes is moving with velocity of 45 Kmph. What is its Kinetic Energy? ii) Define potential energy and state conservation of energy with an example. |  |