## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engg.

Subject: Automobile Electrical Equipments(4063) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Sketch a neat labelled diagram of a spark plug. |  |
| b) | Compare insulated return system with earth return system with respect to their advantages and disadvantages. |  |
| c) | Differentiate between positive and negative earthing systems. |  |
| d) | State the function of the field system in a dynamo. |  |
| e) | Explain the Ampere-hour rating of a lead acid battery. |  |
| f) | Differentiate between a fog lamp and a head lamp. |  |
| g) | Explain the specific gravity test on a battery. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain the construction of a lead-acid battery. |  |
| b) | Explain the various causes of lead acid battery failure. |  |
| c) | Explain the construction of any one type of an alkaline battery. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the maintenance procedure of a lead acid battery. |  |
| b) | Explain the advantages of an alkaline battery over a lead acid battery. |  |
| c) | Explain the different types of cables used in a car. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the operation of a three unit regulator. |  |
| b) | With a neat sketch explain the principle of operation of a dynamo. |  |
| c) | Explain the advantages of an alternator over a dynamo. |  |
| Q.No.5. | Write notes on any two: | 2 x 6 = 12 |
| a) | Selenium Rectifiers |  |
| b) | Motor- dynamo |  |
| c) | Electronic ignition system |  |
| Q.No.6. | Write notes on any two: | 2 x 6 = 12 |
| a) | Bendix drive |  |
| b) | Torque performance curves of a starter motor |  |
| c) | Principle of operation of a starter motor |  |

***********